XGS-1616ST Series

Installation and Getting Started Guide

XGS-1616ST

Installation and Getting Started Guide

16-Port Web Smart Plus 10G Managed Switch

2015, Manufacture Corporation. All rights reserved. All brand and product names are trademarks or registered trademarks of their respective companies

ABOUT THIS GUIDE

PURPOSE
This guide gives specific information on how to operate and use the management functions of the switch.

AUDIENCE 
The guide is intended for use by network administrators who are responsible for operating and maintaining network equipment; consequently, it assumes a basic working knowledge of general switch functions, the Internet Protocol (IP), and Simple Network Management Protocol (SNMP).

CONVENTIONS
The following conventions are used throughout this guide to show information:

WARRANTY

See the Customer Support Warranty booklet included with the product.

A copy of the specific warranty terms applicable to your Manufacture products and replacement parts can be obtained from your Manufacture Sales and Service Office authorized dealer.

|  | NOTE: Emphasizes important information or calls your attention to related features or instructions. |
| --- | --- |
|  | WARNING: Alerts you to a potential hazard that could cause personal injury. |
|  | CAUTION: Alerts you to a potential hazard that could cause loss of data, or damage the system or equipment. |

COMPLIANCES AND SAFETY STATEMENTS

FCC-CLASS A

This equipment has been tested and found to comply with the limits for a Class A computing device pursuant to Subpart J of part 15 of FCC Rules, which are designed to provide reasonable protection against such interference when operated in a commercial environment.

This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

You are cautioned that changes or modifications not expressly approved by the party responsible for compliance could void your authority to operate the equipment.

You may use unshielded twisted-pair (UTP) for RJ-45 connections - Category 5 or better for 100 Mbps connections, Category 5, 5e, or 6 for 1000 Mbps connections, Category 6A or better for 10Gbps Connections. For fiber optic connections, you may use 50/125 or 62.5/125 micron multimode fiber or 9/125 micron single-mode fiber.

CE MARK DECLARATION

OF CONFORMANCE FOR EMI AND SAFETY (EEC)

This equipment has been tested and found to comply with the protection requirements of European Emission Standard EN55022/EN61000-3 and the Generic European Immunity Standard EN55024.

EMC:

| EN55022(2006)+A1:2007/CISPR 22:2006+A1:2006 | Class A 4K V CD, 8KV, AD |
| --- | --- |
| IEC61000-4-2 (2001) | 3V/m |
| IEC61000-4-3( 2002) | 1KV – (power line), 0.5KV – (signal line) |
| IEC61000-4-4(2004) | Line to Line: 1KV, Line to Earth: 2KV |
| IEC61000-4-5 (2001) | 130dBuV(3V) Level 2 |
| IEC61000-4-6 (2003) | 1A/m |
| IEC61000-4-8 (2001) | Voltage dips: >95%, 0.5period, 30%, 25periods |
| IEC61000-4-11(2001) | Voltage interruptions: >95%, 250periods |

|  | CAUTION: Circuit devices are sensitive to static electricity, which can damage their delicate electronics. Dry weather conditions or walking across a carpeted floor may cause you to acquire a static electrical charge. To protect your device, always: - Touch the metal chassis of your computer to ground the static electrical charge before you pick up the circuit device. - Pick up the device by holding it on the left and right edges only. - If you need using outdoor device connect to this device with cable then you need to addition an arrester on the cable between outdoor device and this device. Fig. Addition an arrester between outdoor device and this switch - The switch supports the SFP+ Vendor includes: Manufacture, APAC, Coretek and Avago. |
| --- | --- |

|  | NOTE: The switch is indoor device; if it will be used in outdoor environment or connects with some outdoor device, then it must use a lightning arrester to protect the switch |
| --- | --- |

|  | WARNING: - Self-demolition on Product is strictly prohibited. Damage caused by self-demolition will be charged for repairing fees. - Do not place product at outdoor or sandstorm. - Before installation, please make sure input power supply and product specifications are compatible to each other. - To reduce the risk of electric shock. Disconnect all AC or DC power cord and RPS cables to completely remove power from the unit. - Before importing / exporting configuration please make sure the firmware version is always the same. - After firmware upgrade, the switch will remove the configuration automatically to latest firmware version. |
| --- | --- |

RELATED PUBLICATIONS

The following publication gives specific information on how to operate and use the management functions of the switch:

The User’s Manual

REVISION HISTORY

This section summarizes the changes in each revision of this guide.

| Release | Date | Revision |
| --- | --- | --- |
| 0.82.2 | 10/14/2015 | A1 |
| 0.83.2 | 12/10/2015 | A2 |

Contents

ABOUT THIS GUIDE	ii

COMPLIANCES AND SAFETY STATEMENTS	iii

INTRODUCTION	1

OVERVIEW	1

DESCRIPTION OF HARDWARE	4

NETWORK PLANNING	6

INTRODUCTION TO SWITCHING	6

APPLICATION EXAMPLES	6

INSTALLING THE SWITCH	7

SELECTING A SITE	7

ETHERNET CABLING	7

EQUIPMENT CHECKLIST	8

PACKAGE CONTENTS	8

MOUNTING	8

INSTALLING AN OPTIONAL SFP+ TRANSCEIVER	13

CONNECTING TO A POWER SOURCE	16

Operation of Web-based Management	17

MAKING NETWORK CONNECTIONS	18

CONNECTING NETWORK DEVICES	18

TWISTED-PAIR DEVICES	18

FIBER OPTIC SFP+ DEVICES	20

CABLE LABELING AND  CONNECTION RECORDS	23

TROUBLESHOOTING	24

Basic Troubleshooting Tips	24

POWER AND COOLING PROBLEMS	26

Installation	26

IN-BAND ACCESS	26

SPECIFICATIONS	27

PHYSICAL CHARACTERISTICS	27

SWITCH FEATURES	27

MANAGEMENT FEATURES	28

STANDARDS	28

COMPLIANCES	28

COMPLIANCES	36

INTRODUCTION

OVERVIEW

The XGS-1616ST 10GbE Switch, Manufacture network next generation solutions, is a portfolio of affordable managed switches that provides a reliable infrastructure for your business network. These switches deliver more intelligent features you need to improve the availability of your critical business applications, protect your sensitive information, and optimize your network bandwidth to deliver information and applications more effectively. Easy to set up and use, it provides the ideal combination of affordability and capabilities for entry level Networking includes Small Business or enterprise application and helps you create a more efficient, better-connected workforce.

The XGS-1616ST Switch is broad portfolio of easy-implement managed Ethernet switches. Models include with 16 ports of 10Gigabit Ethernet connectivity, providing ideal flexibility to design suitable network infrastructure for business requirement. However, unlike other entry-level switching solutions that provides advance managed network capabilities only in the costliest models, Switch support the advanced security management capabilities and network features to support includes data, voice, security, and wireless technologies. Besides, these switches are easy to deploy and configure, providing stable and quality performance network services your business needs.

Front of the Switches

Back of the Switches

AC Power Socket

SWITCH ARCHITECTURE

The switch performs a wire-speed, non-blocking switching fabric. This allows wire-speed transport of multiple packets at low latency on all ports simultaneously. The switch also features full-duplex capability on all ports, which effectively doubles the bandwidth of each connection.

This switch uses store-and-forward technology to ensure maximum data integrity. With this technology, the entire packet must be received into a buffer and checked for validity before being forwarded. This prevents errors from being propagated throughout the network.

NETWORK MANAGEMENT OPTIONS

The switch can also be managed over the network with a web browser application.

|  | NOTE: For a detailed description of the management features, refer to the User’s manual. |
| --- | --- |

DESCRIPTION OF HARDWARE

10GBASE-T PORTS

The switch contains 2 10GBASE-T RJ-45 ports. All RJ-45 ports support automatic MDI/MDI-X operation, auto-negotiation and IEEE 802.3x auto-negotiation of flow control, so the optimum data rate and transmission can be selected automatically.

SFP and SFP+ TRANSCEIVER SLOTS

The following table shows a list of transceiver types which have been tested with the switch. For an updated list of vendors supplying these transceivers, contact your local dealer. For information on the recommended standards for fiber optic cabling, see “10Gbps Gigabit Ethernet Collision Domain”.

Supported SFP and SFP+ Transceivers

| Media Standard | Fiber Diameter (microns) | Wavelength (nm) | Maximum Distance* |
| --- | --- | --- | --- |
| 10GBASE-SR | OM3 50/125 | 850 | 300M |
| 1000BASE-SX | 50/125 62.5/125 | 850 850 | 550 m 220 m |
| 1000BASE-LX/ LHX/ XD/ZX | 9/125 9/125 | 1310 1550 | 10,30 km 30,50 km |
| 1000BASE-LX Single Fiber | 9/125 | TX-1310/RX-1550 Tx-1550/RX-1310 | 10,20 km 10,20 km |

|  | NOTE: * Maximum distance may vary for different SFP/SFP+ vendors |
| --- | --- |

PORT AND SYSTEM STATUS LEDS

The XGS-1616ST switch includes a display panel for system and port indications that simplify installation and network troubleshooting. The LEDs, which are located on left hand side of the front panel for easy viewing. Details are shown below and described in the following tables.

Port Status LEDs

| LED | Condition | Status |
| --- | --- | --- |
| TP Speed | Green/Amber | Lit Green when TP link on 10G(Left)/ 1G(Right) Amber when TP link on 100Mbps(Right) |
| SFP+ Speed | Green/Amber | Lit Green when SFP+ link on 10G Amber when SFP+ link on 1G |

System Status LED

| System LED | Condition | Status |
| --- | --- | --- |
| Power | Green OFF | Lit Green when power is coming up |
| System | Green OFF | Lit Green when system is ready |
| Fan | Red | Lit Red when fan is error |

POWER SUPPLY SOCKET

There are a power sockets on the rear panel of the switch. For normal power supply, the XGS-1616ST switch has standard power socket for AC power cord

NETWORK PLANNING

INTRODUCTION TO SWITCHING

A network switch allows simultaneous transmission of multiple packets, and it can partition a network more efficiently than bridges or routers. Therefore the switch has been recognized as one of the most important devices for today’s networking technology.

When performance bottlenecks are caused by congestion at the network access point such as file server, the device can be connected directly to a switched port. And, by using full-duplex mode, the bandwidth of the dedicated segment can be doubled to maximize throughput.

When networks are based on repeater (hub) technology, the distance between end stations is limited by a maximum hop count. However, a switch can subdividing the network into smaller and more manageable segments, and linking them to the larger network than it turns the hop count back to zero and removes the limitation.

A switch can be easily configured in any Ethernet, Fast Ethernet, or Gigabit Ethernet network to significantly increase bandwidth while using conventional cabling and network cards.

APPLICATION EXAMPLES

The XGS-1616ST Switch implements 2 10Gigabit Ethernet TP ports with auto MDIX and 14 slots for the removable SFP+ module which supports comprehensive types of fiber connection, such as LC and BiDi-LC modules. It is not only designed to segment your network, but also to provide a wide range of options in setting up network connections. Some typical applications are described below.

The switch is suitable for the following applications.

- Remote site application is used in Enterprise or SMB

- Peer-to-peer application is used in two remote offices

- Office network

- High Performance Requirement environment

- Advance Security for network safety application

- Suitable for data/ voice and video conference application

INSTALLING THE SWITCH

SELECTING A SITE

The Switch can be mounted in a standard 19-inch equipment rack (Via Optional Rack mount Kit) or on a flat surface. Be sure to follow the guidelines below when choosing a location.

- The site should:

  - Be at the center of all the devices you want to link and near a power outlet.

  - Be able to maintain its temperature within -10 to 50°C (14 to 122 °F) and its humidity within 10% to 90%, non-condensing.

  - Be accessible for installing, cabling and maintaining the devices.

  - Allow the status LEDs to be clearly visible.

- Make sure the twisted-pair Ethernet cable is always routed away from power lines, radios, transmitters or any other electrical interference.

- Make sure that XGS-1616ST Switch is connected to a separate grounded power outlet that provides 100 to 240 VAC, 50 to 60 Hz.

ETHERNET CABLING

To ensure proper operation when installing the switch into a network, make sure that the current cables are suitable for 1000BASE-T or 10GBASE-T operation. Check the following criteria against the current installation of your network:

- Cable type: Unshielded twisted pair (UTP) or shielded twisted pair (STP) cable with RJ-45 connectors; Category 6 or 6a with maximum length of 100 meters is recommend 1000BASE-TX, and Category 6a with maximum length of 100 meters is recommend for 10GBASE-T.

- Protection from radio frequency interference emissions.

- Electrical surge suppression.

- Separation of electrical wires and data based network wiring.

- Safe connections with no damaged cables, connectors or shields.

RJ-45 Connections

SFP+ Transceiver

EQUIPMENT CHECKLIST

After unpacking this switch, please check the contents to be sure you have received all the components. Then, before beginning the installation, be sure you have all other necessary installation equipment.

PACKAGE CONTENTS

- XGS-1616ST 10GbE Management Switch

- Four adhesive rubber feet

- Mounting Accessory (for 19” Rack Shelf, Optional)

- This Installation Guide

- User’s Manual CD

- AC Power Cord

|  | NOTE: Please notify your sales representative immediately if any of the aforementioned items is missing or damaged. |
| --- | --- |

|  | WARNING: The mini-GBICs are Class 1 laser devices. Avoid direct eye exposure to the beam coming from the transmit port. |
| --- | --- |

MOUNTING

The switch can be mounted in a standard 19-inch equipment rack or on a desktop or shelf. Mounting instructions for each type of site as follow.

RACK MOUNTING

Before rack mounting the switch, please pay attention to the following factors:

- Temperature: Since the temperature within a rack assembly may be higher than the ambient room temperature, check that the rack-environment temperature is within the specified operating temperature range (-10 to 50 °C).

- Mechanical Loading: Do not place any equipment on top of a rack-mounted unit.

- Circuit Overloading: Be sure that the supply circuit to the rack assembly is not overloaded.

- Grounding: Rack-mounted equipment should be properly grounded.

TO Rack-mount Devices:

Step1. Attach the brackets to the device using the screws provided in the Mounting Accessory.

Attaching the Brackets

Step2. Mount the device in the rack (Via Optional Rack-Mount kit), using three rack-mounting screws (not provided). Be sure to secure the lower rack-mounting screws first to prevent the brackets being bent by the weight of the switch.

Installing the switch in a Rack

Step3. If installing a single switch only, turn to “Connection to a Power Source” at the end of this chapter.

Step4. If installing multiple switches, mount them in the rack, one below the other, in any order.

Wall-mount Devices:

Step1. Drill two holes (2.7mm in depth) through the wall with the drill (4.4mm to 6.8mm in diameter)

Step2. Place the plastic conical anchors into these two holes, then fasten the screws

Installing the switch on the wall

|  | NOTE: - Allow a proper space for ventilation. - Do not bend the fiber cable vertical at random. - Please notice that the length of the power cord (around 6 feets or 1.8 meters) can match the distance between the converter and power outlet. |
| --- | --- |

DESKTOP OR SHELF MOUNTING:

Step1. Attach the four adhesive rubber feet to the bottom of the first switch.

Attaching the Adhesive Rubber Feet

Step2. Set the device on a flat surface near an AC power source, making sure there are at least two inches of space on all sides for proper air flow.

Step3. If installing a single switch only, go to “Connecting to a Power Source” at the end of this Chapter.

Step4. If installing multiple switches, attach four adhesive feet to each one. Place each device squarely on top of the one below, in any order.

INSTALLING AN OPTIONAL SFP+ TRANSCEIVER

You can install or remove a mini-GBIC SFP+ from a mini-GBIC slot without having to power off the switch. Use only Manufacture mini-GBIC.

|  | NOTE: - The mini-GBIC ports operate only at full duplex. Half duplex operation is not supported. - Ensure the network cable is NOT connected when you install or remove a mini-GBIC. |
| --- | --- |

|  | CAUTION: Use only supported genuine Manufacture mini-GBICs with your switch. Non-Manufacture mini-GBIC might have compatible issue, and their use may result in product malfunction. |
| --- | --- |

Inserting an SFP+ Transceiver into a Slot

The SFP slots support the following optional SFP transceivers:

| Model Name | Description |
| --- | --- |
| SFP.10G.SR | 10GBase-SR 10GE SFP+ Fiber Module, LC Multi-Mode 850nm |
| SFP.LC | 1000Base-SX GE SFP Fiber Module, LC Multi-Mode 850nm |
| SFP.LC.M2 | 1000Base-SX GE SFP Fiber Module, LC Multi-Mode 1310nm 2km |
| SFP.LC.S10 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 10km |
| SFP.LC.S30 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 30km |
| SFP.LC.S50 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 50km |
| SFP.L5.S50 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 50km |
| SFP.BL5.S10 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 10km, 1310nm |
| SFP.BL3.S10 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 10km, 1550nm |
| SFP.BL5.S20 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 20km, 1550nm |
| SFP.BL3.S20 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 20km, 1310nm |

|  | CAUTION: We recommend the SFP+ Transceiver from the following vendors: - Manufacture - Coretech Corporation - AVAGO Technologies - APAC Technologies |
| --- | --- |

TO Install a SFP+ transceiver, do the following:

Step1. Consider network and cabling requirements to select an appropriate SFP+ transceiver type.

Step2. Insert the transceiver with the optical connector facing outward and the slot connector facing down. Note that SFP+ transceivers are keyed so they can only be installed in one orientation.

Step3. Slide the SFP+ transceiver into the slot until it clicks into place.

|  | NOTE: SFP+ transceivers are not provided in the switch package. |
| --- | --- |

CONNECTING TO A POWER SOURCE

You can plug or remove power cord from AC power socket, to switch the power on and off.

Inserting the Power Cord to AC Power Socket

Step1. Insert the power cable plug directly into the AC Socket located at the back of the switch.

Step2. Plug the other end of the cable into a grounded, 3-Pin, AC power source.

Step3. Check the front-panel LEDs as the device is powered on to be sure the SYSTEM LED is lit. If not, check that the power cable is correctly plugged in.

|  | WARNING: For International use, you may need to change the AC line cord. You must use a line cord set that has been approved for the socket type in your country. |
| --- | --- |

- Operation of Web-based Management

The default values of the managed switch are listed in the table below:

| IP Address | 192.168.1.1 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.254 |
| Username | admin |
| Password |  |

To access the web management of XGS-1616ST switch enter the default IP Address in web browser and hit enter. E.g http://192.168.1.1

Once you have entered the IP Address into the web browser you will be prompted to enter a Username and Password in order to access the web management interface. Enter the default values as shown in the table above.

|  | NOTE: For full configuration details of the XGS-1616ST switch please refer to the User Manual |
| --- | --- |

MAKING NETWORK CONNECTIONS

CONNECTING NETWORK DEVICES

The switch is designed to be connected to 100M, 1G, 10Gbps network cards in PCs and servers, as well as to other switches and hubs. It may also be connected to remote devices using optional SFP+ transceivers.

TWISTED-PAIR DEVICES

Each device requires an unshielded twisted-pair (UTP) cable with RJ-45 connectors at both ends. Use Category 6 or 6a cable for 10GBASE-T connections, Category 6 or better for 1000BASE-TX connections.

CABLING GUIDELINES

The RJ-45 ports on the switch support automatic MDI/MDI-X pinout configuration, so you can use standard straight-through twisted-pair cables to connect to any other network device (PCs, servers, switches, routers, or hubs).

|  | CAUTION: Do not plug a phone jack connector into an RJ-45 port. This will damage the switch. Use only twisted-pair cables with RJ-45 connectors that conform to FCC standards. |
| --- | --- |

CONNECTING TO PCS, SERVERS, HUBS AND SWITCHES

Step1. Attach one end of a twisted-pair cable segment to the device’s RJ-45 connector.

Making Twisted-Pair Connections

Step2. If the device is a network card and the switch is in the wiring closet, attach the other end of the cable segment to a modular wall outlet that is connected to the wiring closet. (See the section “Network Wiring Connections.”) Otherwise, attach the other end to an available port on the switch.

Make sure each twisted pair cable does not exceed 100 meters (328 ft) in length.

|  | NOTE: Avoid using flow control on a port connected to a hub unless it is actually required to solve a problem. Otherwise back pressure jamming signals may degrade overall performance for the segment attached to the hub. |
| --- | --- |

NETWORK WIRING CONNECTIONS

Today, the punch-down block is an integral part of many of the newer equipment racks. It is actually part of the patch panel. Instructions for making connections in the wiring closet with this type of equipment follows.

Step1. Attach one end of a patch cable to an available port on the switch, and the other end to the patch panel.

Step2. If not already in place, attach one end of a cable segment to the back of the patch panel where the punch-down block is located, and the other end to a modular wall outlet.

Step3. Label the cables to simplify future troubleshooting. See “Cable Labeling and Connection Records”.

Network Wiring Connections

FIBER OPTIC SFP+ DEVICES

An optional 10Gigabit SFP+ transceiver can be used for a backbone connection between switches, or for connecting to a high-speed server.

Each single-mode fiber port requires 9/125 micron single-mode fiber optic cable with an LC connector at both ends. Each multimode fiber optic port requires 50/125 or 62.5/125 micron multimode fiber optic cabling with an LC connector at both ends.

|  | WARNING: This switch uses lasers to transmit signals over fiber optic cable. The lasers are inherently eye safe in normal operation. However, user should never look directly at a transmit port when it is powered on. WARNING: When selecting a fiber SFP+ device, considering safety, please make sure that it can function at a temperature that is not less than the recommended maximum operational temperature of the product. You must also use an approved Laser SFP+ transceiver. |
| --- | --- |

Step1. Remove and keep the LC port’s rubber plug. When not connected to a fiber cable, the rubber plug should be replaced to protect the optics.

Step2. Check that the fiber terminators are clean. You can clean the cable plugs by wiping them gently with a clean tissue or cotton ball moistened with a little ethanol. Dirty fiber terminators on fiber optic cables will impair the quality of the light transmitted through the cable and lead to degraded performance on the port.

Step3. Connect one end of the cable to the LC port on the switch and the other end to the LC port on the other device. Since LC connectors are keyed, the cable can be attached in only one orientation.

Making Fiber Port Connections

Step4. As a connection is made, check the Link LED on the switch corresponding to the port to be sure that the connection is valid.

CONNECTIVITY RULES

When adding hubs to your network, please note that because switches break up the path for connected devices into separate collision domains, you should not include the switch or connected cabling in your calculations for cascade length involving other devices.

10GBASE-T CABLE REQUIREMENTS

All Category 6a UTP cables that are used for 1000BASE-TX connections should also work for 10GBASE-T, providing that all four wire pairs are connected. However, it is recommended that for all critical connections, or any new cable installations, Category 6 or Category 6a cable should be used. The Category 6 and 6a specifications include test parameters that are only recommendations for Category 6a. Therefore, the first step in preparing existing Category 6a cabling for running 10GBASE-T is a simple test of the cable installation to be sure that it complies with the IEEE 802.3-2005 standards

10GBPS GIGABIT ETHERNET COLLISION DOMAIN

Maximum 10GBASE-T Gigabit Ethernet Cable Length

| Cable Type | Maximum Cable Length | Connector |
| --- | --- | --- |
| Category 6a 100-ohm UTP or STP | 100.m (328 ft) | RJ45 |

Maximum 1000BASE-T Gigabit Ethernet Cable Length

| Cable Type | Maximum Cable Length | Connector |
| --- | --- | --- |
| Category 5, 5e or 6 100-ohm UTP or STP | 100.m (328 ft) | RJ45 |

Maximum 10GBASE-SR Gigabit Fiber Cable Lengths

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| OM3 50/125 micron multimode fiber | 1500 MHz/km | 300 m (984 ft) | LC |

Maximum 1000BASE-SX Gigabit Fiber Cable Lengths

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| 50/125 micron multimode fiber | 400 MHz/km 500 MHz/km | 500 m (1641 ft) 550 m (1805 ft) | LC LC |
| 62.5/125 micron multimode fiber | 160 MHz/km 200 MHz/km | 220 m (722 ft) 275 m (902 ft) | LC LC |

Maximum 1000BASE-LX/LHX/XD/ZX Gigabit Fiber Cable Length

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| 9/125 micron single-mode fiber 1310nm | N/A | 10km (6.2 miles) | LC |
| 9/125 micron single-mode fiber 1550nm | N/A | 30km (18.64 miles) 50km (31.06 miles) | LC LC |

Maximum 1000BASE-LX Single Fiber Gigabit Fiber Cable Length

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| Single-mode TX-1310nm RX-1550nm | N/A | 20km (12.42miles) | BIDI LC |
| Single-mode TX-1550nm RX-1310nm | N/A | 20km (12.42miles) | BIDI LC |

CABLE LABELING AND  CONNECTION RECORDS

When planning a network installation, it is essential to label the opposing ends of cables and to record where each cable is connected. This will allow user to easily locate inter-connected devices, isolate faults and change your topology without need for unnecessary time consumption.

To best manage the physical implementations of your network, follow these guidelines:

- Clearly label the opposing ends of each cable.

- Using your building’s floor plans, draw a map of the location of all network-connected equipment. For each piece of equipment, identify the devices to which it is connected.

- Note the length of each cable and the maximum cable length supported by the switch ports.

- For ease of understanding, use a location-based key when assigning prefixes to your cable labeling.

- Use sequential numbers for cables that originate from the same equipment.

- Differentiate between racks by naming accordingly.

- Label each separate piece of equipment.

- Display a copy of your equipment map, including keys to all abbreviations at each equipment rack.

TROUBLESHOOTING

Basic Troubleshooting Tips

Most problems are caused by the following situations. Check for these items first when starting your troubleshooting:

- Connecting to devices that have a fixed full- duplex configuration.

The RJ-45 ports are configured as “Auto”, that is, when connecting to the attached devices, the switch will operate in one of two ways to determine the link speed and the communication mode (half duplex or full duplex):

    - If the connected device is also configured to Auto, the switch will automatically negotiate both link speed and communication mode.

    - If the connected device has a fixed configuration, for example 10Gbps, at half or full duplex, the switch will automatically sense the link speed, but will default to a communication mode of half duplex.

Because the XGS-1616ST switch devices behave in this way (in compliance with the IEEE802.3 standard), if a device connected to the switch has a fixed configuration at full duplex, the device will not connect correctly to the switch. The result will be high error rates and very inefficient communications between the switch and the device.

Make sure all devices connected to the XGS-1616ST Switch devices are configured to auto negotiate, or are configured to connect at half duplex (all hubs are configured this way, for example).

- Faulty or loose cables. Look for loose or obviously faulty connections. If they appear to be OK, make sure the connections are snug. If that does not correct the problem, try a different cable.

- Non-standard cables. Non-standard cables may cause network collisions and other network problems, and can seriously impair network performance. Use a new correctly-wired cable. For pinouts and correct cable wiring. A category 6a cable tester is a recommended tool for every 1000Base-TX and 10GBase-T network installation.

- Improper Network Topologies. It is important to make sure you have a valid network topology. If you no longer experience the problems, the new topology is probably at fault. In addition, you should make sure that your network topology contains no data path loops.

Troubleshooting Chart

| Symptom | Action |
| --- | --- |
| System LED is Off | - Check connections between the switch, the power cord and the wall outlet. - Contact your dealer for assistance. |
| Link LED is Off | - Verify that the switch and attached device are powered on. - Be sure the cable is plugged into the switch and corresponding device. - If the switch is installed in a rack, check the connections to the punch-down block and patch panel. - Verify that the proper cable type is used and its length does not exceed specified limits. - Check the adapter on the attached device and cable connections for possible defects. Replace the defective adapter or cable if necessary. |

POWER AND COOLING PROBLEMS

Installation

If the System indicator does not turn on when the power cord is plugged in, you may have a problem with the power outlet, power cord, or internal power supply. However, if the unit powers off after running for a while, check for loose power connections, power losses or surges at the power outlet. If you still cannot isolate the problem, the internal power supply may be defective. Verify that all system components have been properly installed. If one or more components appear to be malfunctioning (such as the power cord or network cabling), test them in an alternate environment where you are sure that all the other components are functioning properly.

IN-BAND ACCESS

You can access the management agent in the switch from anywhere within the attached network using a web browser. However, you must first configure the switch with a valid IP address, subnet mask, and default gateway. If you have trouble establishing a link to the management agent, check to see if you have a valid network connection. Then verify that you entered the correct IP address. Also, be sure the port through which you are connecting to the switch has not been disabled. If it has not been disabled, then check the network cabling that runs between your remote location and the switch.

SPECIFICATIONS

PHYSICAL CHARACTERISTICS

PORTS

2 100M/1G/10Gbps TP

14 1G/10G bps SFP+ Fiber ports

NETWORK INTERFACE

Ports 15-16: RJ-45 connector, auto MDI/X

100BASE-TX: RJ-45 (100-ohm, UTP cable; Category 5 or better)

1000BASE-T: RJ-45 (100-ohm, UTP or STP cable; Category 5, 5e or 6)

10GBASE-T: RJ-45 (100-ohm, UTP or STP cable; Category 6a)

*Maximum Cable Length - 100 m (328 ft)

Ports 1-14: 1G/10G SFP+

BUFFER ARCHITECTURE

2MB on-chip frame buffer

AGGREGATE BANDWIDTH

200 Gbps

SWITCHING DATABASE

16K MAC address entries

LEDS

TP Port: status (LINK/ACT), 100M/1G/10G

SFP+ Port: status (LINK/ACT/SPD), 1G/10G

WEIGHT

1.9 kg (4.2 lbs)

SIZE

44(H) x 220(W) x 243(D)mm

TEMPERATURE Operating: -10°C to 50°C (14 to 122 °F)

HUMIDITY

Operating: 10% to 90% (non-condensing)

POWER INPUT

100~240VAC, 50~60Hz

POWER CONSUMPTION

49.4 Watts maximum

SWITCH FEATURES

FORWARDING MODE

Store-and-forward

THROUGHPUT

148.8 Mpps

FLOW CONTROL

Full Duplex: IEEE 802.3x

Half Duplex: Back pressure

MANAGEMENT FEATURES

STANDARDS

I

IEEE 802.3u => 100Base-TX Ethernet (Twisted-pair Copper)

IEEE 802.3ab => 1000Base-TX Ethernet (Twisted-pair Copper)

IEEE 802.3an => 10GBase-T Ethernet

IEEE 802.3x => Flow Control Capability

ANSI/IEEE 802.3 => Auto-negotiation

IEEE 802.1Q => VLAN

IEEE 802.1p => Class of Service

IEEE 802.1X => Access Control

IEEE 802.1D => Spanning Tree

IEEE 802.1w => Rapid Spanning Tree

IEEE 802.1s => Multiple Spanning Tree

IEEE 802.3ad => ink Aggregation Control Protocol (LACP)

IEEE 802.1AB => Link Layer Discovery Protocol (LLDP)

COMPLIANCES

EMISSIONS

EN55022 (CISPR 22) Class A EN 61000-3

FCC Class A

CE Mark

IMMUNITY

EN 61000-4-2/3/4/5/6/8/11

EN 55024