GS-6528XB L2+ Managed GbE SDN Switch

Overview

GS-6528XB L2+ Managed SDN Switch is an innovative SDN/Legacy hybrid mode switch, which is capable of running full suite of L2 features, including basic L3 features such as DHCP Server, better functionality and usability that delivers better cost performance and lower total cost of ownership in Enterprise networks via fiber or copper connections, and at the same time, provides next-generation SDN functions to the network. By collaborating with Network Controller, GS-6528XB can provide efficient SDN networking management functions and dynamic resource adjustment functions to the network, which makes it suitable for those networks that require efficient management tools and flexible resource re-assignment in short time, e.g., IoT networks and smart city /school infrastructure networks.

GS-6528XB is Software Defined Networking (SDN) ready and is able to support OpenFlow v1.3. It is suitable for networks from a small workgroup to a large business either as edge devices in an SDN or be standalone since Hybrid Mode is supported.

GS-6528XB delivers 24 (10M/100M/1G) RJ45, 4 SFP+ ports, 1 USB port, and RJ45 console port. GS-6528XB provides high HW performance and environment flexibility for SMBs and Enterprises.

Key Features

- SDN OpenFlow v1.3 (support Hybrid Mode)

- L2+ Managed features provide easier manageability, robust security and QoS.

- Support Secure boot function (option by request)

- IPv4/IPv6 Management

- IEEE 802.3az EEE Energy Efficient Ethernet standard for green Ethernet

- ITU-T G.8032 Ethernet Ring Protection Switching (ERPS) (to be launched by 2022)

Benefits

- Software Defined Networking (SDN)

OpenFlow v1.3 is a key technology that enables the use of SDN to build smart applications that unlock value and reduce cost. OpenFlow v1.3 support encryption, connection interruption and inactivity probe. It can be managed by any OpenFlow-based SDN Controller, and support hybrid pipeline for legacy L2 features at the same time for more enterprise applications. OpenFlow-hybrid switches support both OpenFlow operation and normal Ethernet switching operation.

- Feature-rich Ethernet Switch for Enterprise-class

The switch delivers advanced functionality in L2+ managed switch. It also has comprehensive security features such as IP source guard and ACL to guard your network from unauthorized access. It helps users to build on the market-leading price/performance with L2+ Managed GbE switch, and provide secure, reliable and ease of use for enterprise/SMB deployments.

- Energy-efficient Design

It is designed to help customers to reduce power consumption and lower the TCO by Energy Efficient Ethernet (IEEE 802.3az) features. It can be used for customers to build a green Ethernet networking environment.

Specifications

Port Configuration

| Total Ports | RJ45 (10M/100M/1G) | Uplinks (1/10G) | Console |
| --- | --- | --- | --- |
| 28 | 24 | 4 SFP+ | RJ45 |

Hardware Performance

| Forwarding Capacity (Mpps) | Switching Capacity (Gbps) | Mac Table (K) | Jumbo Frames (Bytes) |
| --- | --- | --- | --- |
| 95.232 | 128 | 32 | 12K |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Operating Humidity | Altitude |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | 10% to 90% non-condensing | Feet | Meters |
| 32 to 122 | 0 to 50 | -4 to 158 | -20 to 70 |  | < 10000 | <3000 |

Dimension, Weights, Mounting

| Dimension (WxHxD) |  | Weight |  | Mounting Type |
| --- | --- | --- | --- | --- |
| Millimeter | Inches | Kilograms | Pounds |  |
| 442x 44x 211 | 17.4x 1.7x 8.3 | <3.8 | <8.36 | Desktop, Rack |

Voltage and Frequency

| AC Input Voltage and Frequency |  |  |  |
| --- | --- | --- | --- |
| Voltage | 100-240 VAC | Frequency | 50~60 Hz |

Certifications

Electromagnetic Emissions (EMC)

EN61000-4-2, EN61000-4-3, EN61000-4-6, EN61000-4-4, EN61000-4-5

CE, FCC Part 15 Class A, LVD (UL by request)

Software Features

| SDN OpenFlow Features |  |  |
| --- | --- | --- |
| Version |  | V1.3 |
| Hybrid Mode |  | Support |
| Classification Mechanism |  | Port/VLAN/Port+Vlan |
| Pipeline Traversal |  | - OpenFlow pipeline - Traditional pipeline |
| Entry Table |  | 32K |
| Flow Table Stage |  | 5 (3 Full Match+ L2 Optimized+ L3 Optimized, Loopback extensible) |
| Entry Capability |  | - Operation - Hit Flag |
| Metadata Match |  | Support |
| Group Table |  | - 2K Group Entries - 8K Action Buckets |
| Meter Table |  | - 512 policer - Drop |
| Counter |  | - 4K 32-bit packet counter - 4K 64-bit byte counter |
| Layer 2 Switching |  |  |
| Spanning Tree Protocol (STP) |  | - Standard Spanning Tree 802.1d - Rapid Spanning Tree (RSTP) 802.1w - Multiple Spanning Tree (MSTP) 802.1s - BPDU Guard - STP Root Guard |
| Trunking |  | Link Aggregation Control Protocol (LACP) IEEE 802.3ad - Up to 26 groups - Up to 16 ports per group |
| VLAN |  | Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs) - Port-based VLAN - 802.1Q tag-based VLAN - MAC-based VLAN - Management VLAN - Private VLAN - Guest VLAN - Voice VLAN - 802.1v Protocol VLAN - IP Subnet-Based VLAN |
| GARP VLAN Registration Protocol (GVRP) |  | GVRP stands for GARP (Generic Attribute Registration Protocol) VLAN Registration Protocol. It’s a Layer 2 network protocol, for automatic configuration of switches in a VLAN network |
| IGMP v1/v2/v3 Snooping |  | IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups |
| MLD v1/v2 Snooping |  | Delivers IPv6 multicast packets only to the required receivers |
| Multicast VLAN Registration (MVR) |  | It uses a dedicated manually configured VLAN, called the multicast VLAN, to forward multicast traffic over Layer 2 network in conjunction with IGMP snooping. |
| L3 Features |  |  |
| DHCP Server | Support DHCP server to assign IP to DHCP clients (to be launched by 2022) |  |
| Security |  |  |
| Secure Shell (SSH) |  | SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported |
| Secure Sockets Layer (SSL) |  | SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch |
| IEEE 802.1X |  | - IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions - Dynamic VLAN assignment |
| Port Security |  | Locks MAC addresses to ports, and limits the number of learned MAC address |
| TACACS+ |  | Supports TACACS+ authentication. |
| Storm Control |  | Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port |
| ACLs |  | Supports up to 256 entries. Drop or rate limitation based on: - Source and destination MAC, VLAN ID or IP address, port |
| IP Source guard |  | Prevents illegal IP address from accessing to specific port in the switch |
| DHCP Snooping |  | A feature acts as a firewall between untrusted hosts and trusted DHCP servers |
| Loop Protection |  | To prevent unknown unicast, broadcast and multicast loops in Layer 2 switching configurations. |
| DHCP Client |  | Support DHCP server to assign IP to DHCP clients |
| DHCP Relay (option 66,option 67,option 82) |  | - Relay of DHCP traffic to DHCP server in different VLAN. - Works with DHCP Option 82 |
| Quality of Service |  |  |
| Hardware Queue |  | Supports 8 hardware queues |
| Scheduling |  | - Strict priority and weighted round-robin (WRR) - Queue assignment based on DSCP and class of service |
| Classification |  | - Port based - 802.1p VLAN priority based - IPv4/IPv6 precedence / DSCP based - Classification and re-marking TCP/IP ACLs: QoS by ACL |
| Rate Limiting |  | - Ingress policer - Egress shaping and rate control |
| Management |  |  |
| Remote Monitoring (RMON) |  | Embedded RMON agent supports RMON groups 1,2,3,9 (history, statistics, alarms, and events) for enhanced traffic management, monitoring and analysis |
| Port Mirroring |  | Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported. |
| Web GUI Interface |  | Built-in switch configuration utility for browser-based device configuration |
| CLI/ Telnet |  | For users to configure/manage switches in command line modes |
| Dual Image |  | Independent primary and secondary images for backup while upgrading |
| SNMP |  | SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM) |
| Firmware Upgrade |  | - Web browser upgrade (HTTP/ HTTPs) and TFTP, SFTP, and SCP - Upgrade through console port as well |
| Syslog |  | Support syslog function |
| SMTP |  | Support SMTP (RFC821) |
| IEEE 802.1ab (LLDP) |  | - Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network - Support LLDP-MED extensions |
| CDP Aware |  | The CDP operation is restricted to decoding incoming CDP frames (The switch doesn’t transmit CDP frames). CDP frames are only decoded if LLDP on the port is enabled |
| sFlow |  | The industry standard for monitoring high speed switched networks. It gives complete visibility into the use of networks enabling performance optimization, accounting/billing for usage, and defense against security threats |
| NTP |  | Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched |
| Other Management |  | IPv6 Management |