
IPGS-2612B 

Industrial L2+ Managed GbE PoE+ Switch
[image: image1.jpg]System LEDs

10/100/1000 RJ45 Ports — DC 54V Power Input

Digital Input

Digital Output Relay

Ports Status LEDs
Console Port
100/1000 SFP Ports —| . . .
DIP Switch (Ring Settings)
Reset Button

Ground Screw




Overview
IPGS-2612B industrial L2+ managed GbE PoE+ switch is the next generation industrial grade Ethernet switch offering powerful L2 and basic L3 features with better functionality and usability. In addition to the extensive management features, of which make them suitable for industrial applications.
IPGS-2612B delivers 8 (10M/100M/1G) RJ45 with 8 PoE+ (Support 802.3 at/af, and total up to 240W) ports, 4 GbE SFP ports and RJ45 console port. IPGS-2612B provides high HW performance and environment flexibility for industrial applications. 

The embedded Device Managed System (DMS) features provides users with the benefits of easy-to-use/configure/install/troubleshoot in the video surveillance, wireless access, and other industrial applications. IPGS-2612B is ideal to deliver management simplicity, better user experience, and lowest total cost of ownership.
Key Features
· Rapid Ring (R-Ring)
· Built in Device Management System (DMS)

· iPush APP for real time alarm notification
· DHCP Server
· IEEE 802.3az Energy Efficient Ethernet standard for green Ethernet application

· IEEE 802.3af/at Power over Ethernet

· IPv4/IPv6 L3 static route
· EtherNet/IP (by request)
· PROFINET (by request)
Benefits 

· Feature-rich Specifications to Support Various Applications
The switch deliver extensive industrial grade functionalities, including R-Ring. It also have enhanced L2/L3 features for better manageability and usability. 

It provides advanced PoE features such as PoE auto-checking, PoE scheduling, and PoE power delay for users to manage the powered devices more easily. With extensive PoE+ mode configured, it can even provides power with up to 38W per port.

It offers users with better price/performance ratio in industrial application, and provide secure and reliable functionalities for metro Ethernet deployments.
· Easy to Install, Configure and Troubleshoot by Device Management System
The DMS provides embedded functions to facilitate devices management at anytime and anywhere. Its user-friendly interface helps users to manage devices intuitively. 

It supports various IP device types (e.g. PC, IP-phone, IP-camera, WiFi-AP) for end users to enhance manageability and save time/cost during installation/maintenance stages.
· Lowing Total Cost of Ownership (TCO) with Energy-efficient Design

The switch is designed to help companies to save power needs and reduce TCO by Energy Efficient Ethernet (IEEE 802.3az). It can be used for customers to build a green Ethernet networking environment.

Specifications
Port Configuration

	Total Ports
	RJ45 (10M/100M/1G)
	Uplinks (100M/1G)
	Console
	Ring Mgmt.
	DI/DO

	12
	8
	4 SFP
	RJ45
	DIP
	1/1


Hardware Performance

	Forwarding 
Capacity (Mpps)
	Switching 
Capacity (Gbps)
	Mac Table
(K)
	Jumbo Frames
(Bytes)

	17.856
	24
	8
	9216


Environmental Range
	Operating Temperature
	Storage Temperature
	Operating Humidity
	Altitude

	Fahrenheit
	Centigrade
	Fahrenheit
	Centigrade
	5% to 95%  
non-condensing
	Feet
	Meters

	-40 to 167
	-40 to 75 
	-40 to 185
	-40 to 85
	
	< 10000
	<3000


Dimension, Weights, Mounting
	Dimension (WxHxD)
	Weight
	Mounting Type

	Millimeter
	Inches
	Kilograms
	Pounds
	

	62x 135x 130
	2.4x 5.3x 5.1
	< 1 
	<2.2
	DIN rail


Voltage and Frequency
	Primary Power Supply - DC Input Voltage

	DC Nominal
	54 VDC dual inputs

	DC Operating Range
	48 to 57 VDC

	PoE SKUs
	· Required >48 VDC for PoE IEEE 802.3af (Max. 15.4W) output 
· Required >54 VDC for PoE+ IEEE802.3at (Max. 30W) output 


PoE Power Capacity
	Available PoE Power
	Number of Ports That Support 
PoE(15.4W), PoE+(30.0W)

	240W
	Each of port 1 - 8 support PoE/ PoE+ within available PoE Power

( per port 38W by request )


Certifications
	Regulatory Compliance 

	EMS  
	EN61000-4-2 ESD, EN61000-4-3 RS, EN61000-4-4 EFT, EN61000-4-6 CS, EN61000-4-8 PFMF, 
(EN61000-6-2 by request)

	EMI
	FCC Part 15 Class A 
(EN61000-3-2, EN61000-3-3, EN61000-6-4, EN55022, EN55011 by request)

	Safety
	CE, (EN60950 by request)

	Mechanical Stability

	Vibration
	IEC 60068-2-6

	Shock 
	IEC 60068-2-27

	Freefall 
	IEC 60068-2-32

	Approvals (by request)

	Railway Norm
	EN50121-4, EN50155 

	Transportation
	NEMA TS2 

	Substation
	IEC61850-3, IEEE1613 

	Marine
	DNV


Software Features 

	Ring Management

	Rapid Ring
	Enable self-recover time in less than 20ms

	Device Management System (DMS)

	Graphical 

Monitoring
	· Topology view: Support intuitive way to configure and manage switches and devices with visual relations

· Floor view: It’s easy to drag and drop PoE devices and help you to build smart workforces

· Map view: Enhance efficiency to drag and drop devices and monitor surroundings on google map

	Find my Switch
	Search and manage your real switches quickly.

	Traffic Monitoring
	Display visual chart of network traffic of all devices and monitor every port at any time from switches

	Trouble Shooting
	· Network diagnostic between master switch and devices
· Support protection mechanism, such as rate-limiting to protect your devices from brute-force downloading 

	Industrial Protocols (by Request)

	EtherNet/IP
	EtherNet/IP is an industrial Ethernet network that combines standard Ethernet technologies with the media-independent Common Industrial Protocol.

	PROFINET
	It can be recorded and displayed using an Ethernet analysis tool such as Wireshark. The topology can be shown using analysis tools such as TH Scope.

	Layer 2 Switching 

	Spanning Tree Protocol (STP)
	· Standard Spanning Tree 802.1d 

· Rapid Spanning Tree (RSTP) 802.1w 

· Multiple Spanning Tree (MSTP) 802.1s

	Trunking
	· Link Aggregation Control Protocol (LACP) IEEE 802.3ad

· Up to 6 groups and up to 4 ports per group

	VLAN
	· Port-based VLAN

· 802.1Q tag-based VLAN

· MAC-based VLAN

· Management VLAN

· Private VLAN Edge (PVE)

· Q-in-Q (double tag) VLAN

· Voice VLAN

· GARP VLAN Registration Protocol (GVRP)

	DHCP Relay 
	· Relay of DHCP traffic to DHCP server in different VLAN. 

· Works with DHCP Option 82

	IGMP v1/v2/v3 Snooping
	IGMP limits bandwidth-intensive multicast traffic to only the requesters. 
Supports 1024 multicast groups 

	IGMP Querier
	IGMP querier is used to support a Layer 2 multicast domain of snooping 
switches in the absence of a multicast router

	IGMP Proxy
	IGMP snooping with proxy reporting or report suppression actively filters IGMP packets in order to reduce load on the multicast router

	MLD v1/v2 Snooping
	Delivers IPv6 multicast packets only to the required receivers

	Multicast VLAN Registration (MVR)
	It uses a dedicated manually configured VLAN, called the multicast VLAN, to forward multicast traffic over Layer 2 network in conjunction with IGMP snooping.

	Layer 3 Switching 

	IPv4 Static Routing
	IPv4 Unicast: Static routing

	IPv6 Static Routing
	IPv6 Unicast: Static routing

	Security  

	Secure Shell (SSH)
	SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported

	Secure Sockets Layer (SSL)
	SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch

	IEEE 802.1X
	· IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions

· Supports IGMP-RADIUS based 802.1X

· Dynamic VLAN assignment

	Layer 2 Isolation Private VLAN Edge 
	PVE (also known as protected ports) provides L2 isolation between clients in the same VLAN. Supports multiple uplinks

	Port Security
	Locks MAC addresses to ports, and limits the number of learned MAC address

	IP Source Guard
	Prevents illegal IP address from accessing to specific port in the switch

	RADIUS/ TACACS+ 
	Supports RADIUS and TACACS+ authentication. Switch as a client

	Storm Control
	Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port

	DHCP Snooping
	A feature acts as a firewall between untrusted hosts and trusted DHCP servers

	ACLs
	Supports up to 256 entries. Drop or rate limitation based on:

· Source and destination MAC, VLAN ID or IP address, protocol, port,

· Differentiated services code point (DSCP) / IP precedence

· TCP/ UDP source and destination ports

· 802.1p priority

· Ethernet type

· Internet Control Message Protocol (ICMP) packets

· TCP flag

	Loop Protection
	To prevent unknown unicast, broadcast and multicast loops in Layer 2 switching configurations.

	Quality of Service 

	Hardware Queue 
	Supports 8 hardware queues

	Scheduling
	· Strict priority and weighted round-robin (WRR)

· Queue assignment based on DSCP and class of service

	Classification
	· Port based

· 802.1p VLAN priority based

· IPv4/IPv6 precedence / DSCP based

· Differentiated Services (DiffServ)

· Classification and re-marking ACLs

	Rate Limiting
	· Ingress policer

· Egress shaping and rate control

· Per port

	Management

	HW Monitoring
	Temperature Detection and Alarm

	HW Watchdog
	Supported to resume operation from CPU hang up

	iPush
	· The real time alarm notification could lower technical support cost 

· Works with iOS and Android devices to make quick work of even the most demanding tasks.

	DHCP Server
	Support DHCP server to assign IP to DHCP clients

	Remote Monitoring (RMON)
	Embedded RMON agent supports RMON groups 1,2,3,9 (history, statistics, alarms, and events) for enhanced traffic management, monitoring and analysis

	Port Mirroring
	Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported.  

	UPnP
	The Universal Plug and Play Forum, an industry group of companies working to enable device-to-device interoperability by promoting Universal Plug and Play 

	s-Flow
	The industry standard for monitoring high speed switched networks. It gives complete visibility into the use of networks enabling performance optimization, accounting/billing for usage, and defense against security threats 

	IEEE 802.1ab (LLDP) 
	· Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network

· Support LLDP-MED extensions

	Web GUI Interface
	Built-in switch configuration utility for browser-based device configuration 

	CLI
	For users to configure/manage switches in command line modes

	Dual Image
	Independent primary and secondary images for backup while upgrading 

	SNMP 
	SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM)

	Firmware Upgrade
	· Web browser upgrade (HTTP/ HTTPs) and TFTP

· Upgrade through console port as well

	NTP
	Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched

	Other Management
	· HTTP/HTTPs; SSH

· DHCP Client/ DHCPv6 Client 

· Cable Diagnostics

· Ping

· Syslog

· IPv6 Management

	Power over Ethernet (PoE)

	Port Configuration
	Supports per port PoE configuration function

	PoE Scheduling
	Supports per port PoE scheduling to turn on/off the PoE devices (PDs).

	Auto-checking
	Check the link status of PDs. Reboot PDs if there is no responses.

	Power Delay
	The switch provides power to the PDs based on delay time when PoE switch boots up, in order to protect switch from misuse of the PDs


PAGE  

- 1 -
IPGS-2612B Datasheet

