XGS-1610ST Web Smart+ 10GbE Switch

Overview

XGS-1610ST Web Smart+ Managed Switch is a cost-effective 10GbE Ethernet Switch product line offering 10G copper and fiber connections in a small form factor. With the enormous growth of network traffic and network storage in recent years, 10GbE is becoming a natural upgrade for many businesses in order to keep up with their network performance and business efficiency demands.

XGS-1610ST delivers 2 GbE/10G RJ45 ports and 8 GbE/10G SFP+ ports. It provides higher bandwidth and reliability for SMB and enterprise applications.

XGS-1610ST is ideal to provide connection flexibility across a network allowing easier network integration for virtualization, cloud services and server-to-server applications that expand your enterprise network by adding local switching capacity and better scalability, so that you can support more high-bandwidth applications such as server farm, TV wall and digital signage video streaming, etc.

Key Features

- Web Smart+ features provide easier manageability, basic security and QoS.

- IEEE 802.3az EEE Energy Efficient Ethernet standard for green Ethernet

- 8.6"-wide Small Form Factor Design

- All 10GbE Copper and Fiber Ports

- Auto Fan Control for Better Reliability and Noise Reduction

Benefits

- Cost-effective Ethernet Switch for Enterprise-class

The switch delivers advanced functionality in Web Smart+ managed switch including Layer 3 static route, LLDP, etc. It also has basic security features such as Access Control List, RADIUS and IEEE 802.1x to protect your network from unauthorized access.

It helps users to build on the market-leading price/performance with Web Smart+ managed GbE switch, and provide ease of use for enterprise and SMB deployments.

- Lower Total Cost of Ownership (TCO) with Energy-efficient Design

It is designed to help customers to reduce power consumption and lower the TCO by Energy Efficient Ethernet (IEEE 802.3az) features. It can be used for customers to build a green Ethernet networking environment.

- Small Form Factor

Help you put the switch in locations that might not have sufficient room compared to other 10G switch which required larger space.

- 10GbE Interfaces

Provide superior network performance at non-blocking rate to delivers true 10GbE speed.

- Auto-Negotiation

Senses and adjusts network speed and cabling type automatically.

Specifications

Port Configuration

| Total Ports | Dominant Port |  | Uplink Port |  | Console |
| --- | --- | --- | --- | --- | --- |
|  | RJ45 100M/1G/10G | SFP+ 1G/10G | RJ45 100M/1G/10G | SFP+ 1G/10G |  |
| 10 | -- | 8 | 2 | -- | -- |

Hardware Performance

| Forwarding Capacity | Switching Capacity | Mac Table | Jumbo Frames |
| --- | --- | --- | --- |
| 148.8 Mpps | 200 Gbps | 16 K | 9216 Bytes |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Altitude |  |
| --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | Feet | Meters |
| 14 to 122 | -10 to 50 | -4 to 158 | -20 to 70 | < 10000 | < 3000 |

Dimension, Weights, Humidity

| Dimension (WxHxD) |  |  | Weight |  |  | Operating Humidity |
| --- | --- | --- | --- | --- | --- | --- |
| Millimeter | Inches |  | Kilograms | Pounds |  |  |
| 220 x 44 x 242 | 8.66 x 1.73 x 9.5 | 1.9 |  |  | 4.19 | 10% to 90% non-condensing |

Voltage/Frequency, Acoustic, and Compliance Information

| FAN | Voltage/Frequency | Acoustic | Certification |
| --- | --- | --- | --- |
| 2 | 100~240V / 50~60 Hz | < 42db at 25℃ (refer to ISO 7779) | CE, FCC Class A Others by Request |

Software Features

| Layer 2 Switching |  |
| --- | --- |
| Spanning Tree Protocol (STP) | - Standard Spanning Tree 802.1d - Rapid Spanning Tree (RSTP) 802.1w - Multiple Spanning Tree (MSTP) 802.1s |
| Trunking | - Link Aggregation Control Protocol (LACP) IEEE 802.3ad - Static aggregation |
| VLAN | Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs) - Port-based VLAN - 802.1Q tag-based VLAN - MAC-based VLAN - Private VLAN Edge (PVE) - Protocol-based VLAN - Voice VLAN - Q-in-Q (double tag) VLAN |
| IGMP v1/v2/v3 Snooping | IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups |
| Layer 3 Switching |  |
| ARP | - Static ARP - ARP inspection |
| Security |  |
| IP Source Guard | Prevents illegal IP address from accessing to specific port in the switch |
| Storm Control | Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port |
| ACLs | Supports up to 384 entries. Drop or rate limitation based on: - Source and destination MAC, VLAN ID or IP address, protocol, port - TCP/ UDP source and destination ports - 802.1p priority - Ethernet type - Internet Control Message Protocol (ICMP) packets - TCP flag |
| Quality of Service |  |
| Hardware Queue | Supports 8 hardware queues |
| Scheduling | - Strict priority, weighted round-robin (WRR) and DWRR - Queue assignment based on DSCP and class of service |
| Classification | - Port based - DSCP based |
| Quality of Service |  |
| Rate Limiting | - Ingress policer - Egress shaping and rate control - Per port |
| Management |  |
| Port Mirroring | Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. |
| NTP | Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched |
| DHCP | - DHCP Client - DHCP Snooping |
| IEEE 802.1ab (LLDP) | - Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network - Support LLDP-MED extensions |
| SNMP | SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM) |
| Web GUI Interface | Built-in switch configuration utility for browser-based device configuration |
| Dual Image | Independent primary and secondary images for backup while upgrading |
| Firmware Upgrade | - Web browser upgrade (HTTP/ HTTPs) and TFTP |
| CLI | For users to configure/manage switches in command line modes |
| Other Management | - HTTP/HTTPs - Ping - Syslog |