GS-1626G Web Smart+ GbE Switch

Overview

GS-1626G Web Smart+ Managed Switch is a next-generation Ethernet Switch offering powerful L2 features and Layer 3 Static Route that delivers the cost-effectively business and transports Ethernet services via fiber or copper connections.

GS-1626G delivers 24 (10M/100M/1G) RJ45 ports and 2 Combo GbE RJ45/SFP ports. GS-1626G provides high HW performance and environment flexibility for SMBs and Enterprises.

GS-1626G is ideal to deliver management simplicity, optimum user experience, and lower total cost of ownership. The embedded Device Managed System is designed to be extremely easy-to-use/manage/install IP Phone, IP Cam, or Wifi-AP for Enterprise Applications.

Key Features

- Web Smart+ features provide easier manageability, basic security and QoS

- Built in Device Management System (DMS)

- DHCP Server

- IPv4/IPv6 L3 static route

- IEEE 802.3az EEE Energy Efficient Ethernet standard for green Ethernet

Benefits

- Cost-effective Ethernet Switch for Enterprise-class

The switch delivers advanced functionality in Web Smart+ managed switch including Layer 3 static route, DHCP server, IPv6 support, LLDP, etc. It also has basic security features such as ACL, RADIUS and IEEE 802.1x to protect your network from unauthorized access.

It helps users to build on the market-leading price/performance with Web Smart+ managed GbE PoE switch, and provide ease of use for enterprise and SMB deployments.

- Easy to Install, Configure and Troubleshoot by Device Management System

The DMS provides embedded functions to facilitate devices management at anytime and anywhere. Its user-friendly interface helps users to manage devices intuitively.

It supports various IP device types (e.g. IP-phone, IP-camera, WiFi-AP) for end users to enhance manageability and save time/cost during installation/maintenance stages.

- Lower Total Cost of Ownership (TCO) with Energy-efficient Design

It is designed to help customers to reduce power consumption and lower the TCO by Energy Efficient Ethernet (IEEE 802.3az) features. It can be used for customers to build a green Ethernet networking environment.

Specifications

Port Configuration

| Total Ports | RJ45 (10M/100M/1G) | Uplinks (100M/1G) | Console |
| --- | --- | --- | --- |
| 26 | 24 | 2 RJ45/SFP Combo | -- |

Hardware Performance

| Forwarding Capacity | Switching Capacity | Mac Table | Jumbo Frames |
| --- | --- | --- | --- |
| 38.68 Mpps | 52 Gbps | 32 K | 9216 Bytes |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Altitude |  |
| --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | Feet | Meters |
| 32 to 104 | 0 to 40 | -4 to 158 | -20 to 70 | < 10000. | < 3000 |

Dimension, Weights, Humidity

| Dimension (WxHxD) |  |  | Weight |  |  | Operating Humidity |
| --- | --- | --- | --- | --- | --- | --- |
| Millimeter | Inches |  | Kilograms | Pounds |  |  |
| 442 x 44 x 211 | 17.4 x 1.73 x 8.3 | 2.7 |  |  | 5.9 | 10% to 90% non-condensing |

Voltage and Frequency

| AC Input Voltage and Frequency |  |
| --- | --- |
| Voltage | 100-240 VAC |
| Frequency | 50~60 Hz |

Certification

Electromagnetic Emissions (EMC)

CE, FCC Part 15 Class A

Software Features

| Layer 2 Switching |  |
| --- | --- |
| Spanning Tree Protocol (STP) | - Standard Spanning Tree 802.1d - Rapid Spanning Tree (RSTP) 802.1w - Multiple Spanning Tree (MSTP) 802.1s |
| Trunking | Link Aggregation Control Protocol (LACP) IEEE 802.3ad - Up to 13 groups - Up to 16 ports per group |
| VLAN | Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs) - Port-based VLAN - 802.1Q tag-based VLAN - Management VLAN - Q-in-Q (double tag) VLAN |
| IGMP v1/v2/v3 Snooping | IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups |
| IGMP Querier | IGMP querier is used to support a Layer 2 multicast domain of snooping switches in the absence of a multicast router |
| IGMP Proxy | IGMP snooping with proxy reporting or report suppression actively filters IGMP packets in order to reduce load on the multicast router |
| Device Management System (DMS) |  |
| Graphical Monitoring | - Topology view: Support intuitive way to configure and manage switches and devices with visual relations - Floor view: It’s easy to drag and drop PoE devices and help you to build smart workforces - Map view: Enhance efficiency to drag and drop devices and monitor surroundings on google map |
| Find my Switch | Search your real switches quickly and manage directly. |
| Traffic Monitoring | Display visual chart of network traffic of all devices and monitor every port at any time from switches |
| Trouble Shooting | - Network diagnostic between master switch and devices - Support protection mechanism, such as rate-limiting to protect your devices from brute-force downloading |
| Layer 3 Switching |  |
| IPv4 Static Routing | IPv4 Unicast: Static routing |
| IPv6 Static Routing | IPv6 Unicast: Static routing |

| Security |  |
| --- | --- |
| Secure Shell (SSH) | SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported |
| Secure Sockets Layer (SSL) | SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch |
| IEEE 802.1X | - IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions - Supports IGMP-RADIUS based 802.1X - Dynamic VLAN assignment |
| Layer 2 Isolation Private VLAN Edge | PVE (also known as protected ports) provides L2 isolation between clients in the same VLAN. Supports multiple uplinks |
| Port Security | Locks MAC addresses to ports, and limits the number of learned MAC address |
| Storm Control | Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port |
| Quality of Service |  |
| Hardware Queue | Supports 8 hardware queues |
| Scheduling | - Strict priority and weighted round-robin (WRR) - Queue assignment based on DSCP and class of service |
| Classification | - Port based - 802.1p VLAN priority based - IPv4/IPv6 precedence / DSCP based - Differentiated Services (DiffServ) - Classification and re-marking ACLs |
| Rate Limiting | - Ingress policer - Egress shaping and rate control - Per port |
| Management |  |
| DHCP Server | Support DHCP server to assign IP to DHCP clients |
| Port Mirroring | Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported. |
| UPnP | The Universal Plug and Play Forum, an industry group of companies working to enable device-to-device interoperability by promoting Universal Plug and Play |
| IEEE 802.1ab (LLDP) | - Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network - Support LLDP-MED extensions |
| Web GUI Interface | Built-in switch configuration utility for browser-based device configuration |
| Dual Image | Independent primary and secondary images for backup while upgrading |
| SNMP | SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM) |
| Management |  |
| Firmware Upgrade | Web browser upgrade (HTTP/ HTTPs) and TFTP |
| NTP | Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched |
| Other Management | - HTTP/HTTPs; SSH - DHCP Client/ DHCPv6 Client - Cable Diagnostics - Ping - Syslog - IPv6 Management |