PSGS-2652F L2+ Managed GbE PoE+ Switch

Overview

PSGS-2652F L2+ managed GbE PoE+ switch is a next-generation Ethernet switch offering full suite of L2 features with better PoE functionality and usability, including advanced L3 features such as static route that delivers better cost performance and lower total cost of ownership in Enterprise networks via fiber or copper connections.

PSGS-2652F delivers 50 (10M/100M/1G) RJ45 with 48 PoE+ (Support 802.3at/af, and total up to 740W) ports, 2 GbE SFP ports and RJ45 console port. PSGS-2652F provides high HW performance and environment flexibility for SMBs and Enterprises.

The embedded Device Managed System (DMS) features provides users with the benefits of easy-to-use/configure/install/troubleshoot in the video surveillance, wireless access, and other SMBs and Enterprises applications. PSGS-2652F is ideal to deliver management simplicity, better user experience, and lowest total cost of ownership.

Key Features

- L2+ Managed features provide easier manageability, robust security and QoS.

- Built in Device Management System (DMS)

- DHCP Server

- IPv4/IPv6 Management

- PoE Port configuration and scheduling

- 802.3at high power PoE plus standard

- IEEE 802.3az EEE Energy Efficient Ethernet standard for green Ethernet

Benefits

- Feature-rich Ethernet Switch for Enterprise-class

The switch delivers advanced functionality in L2+ managed switch including Layer 3 static route, DHCP server, IPv6 support, LLDP, etc. It also has comprehensive security features such as IP source guard and ACL to guard your network from unauthorized access.

It helps users to build on the market-leading price/performance with L2+ Managed GbE PoE switch, and provide secure, reliable and ease of use for enterprise/SMB deployments.

- Easy to Install, Configure and Troubleshoot by Device Management System

The DMS provides embedded functions to facilitate devices management at anytime and anywhere. Its user-friendly interface helps users to manage devices intuitively.

It supports various IP device types (e.g. PC, IP-phone, IP-camera, WiFi-AP) for end users to enhance manageability and save time/cost during installation/maintenance stages.

- Lower Total Cost of Ownership (TCO) with Energy-efficient Design

It is designed to help customers to reduce power consumption and lower the TCO by Energy Efficient Ethernet (IEEE 802.3az) features. It can be used for customers to build a green Ethernet networking environment.

- Advanced Power over Ethernet Management

The model includes PoE+ options to power IP devices with power-saving features like Power scheduling and PoE configuration.

Specifications

Port Configuration

| Total Ports | RJ45 (10M/100M/1G) | Uplinks (100M/1G) | Console |
| --- | --- | --- | --- |
| 52 | 50 | 2 SFP | RJ45 |

Hardware Performance

| Forwarding Capacity (Mpps) | Switching Capacity (Gbps) | Mac Table (K) | Jumbo Frames (Bytes) |
| --- | --- | --- | --- |
| 77.376 | 104 | 32 | 10056 |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Operating Humidity | Altitude |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | 10% to 90% non-condensing | Feet | Meters |
| 32 to 104 | 0 to 40 | -4 to 158 | -20 to 70 |  | < 10000. | < 3000 |

Dimension, Weights, Mounting

| Dimension (WxHxD) |  |  | Weight |  |  | Mounting Type |
| --- | --- | --- | --- | --- | --- | --- |
| Millimeter | Inches |  | Kilograms | Pounds |  |  |
| 442 x 44 x 375 | 17.4 x 1.7 x 14.8 | 5.62 |  |  | 12.39 | Desktop, Rack |

Voltage and Frequency

| AC Input Voltage and Frequency |  |
| --- | --- |
| Voltage | 100-240 VAC |
| Frequency | 50~60 Hz |

PoE Power Capacity

| Available PoE Power | Number of Ports That Support PoE(15.4W) and PoE+(30.0W) |
| --- | --- |
| 740W | Each of port 1 - 48 support PoE/ PoE+ within available PoE Power |

Certification

Electromagnetic Emissions (EMC)

CE, FCC Part 15 Class A

Software Features

| Layer 2 Switching |  |
| --- | --- |
| Spanning Tree Protocol (STP) | - Standard Spanning Tree 802.1d: Spanning Tree Protocol is an OSI layer-2 protocol which ensures a loop free topology for any bridged LAN - Rapid Spanning Tree (RSTP) 802.1w: Rapid Spanning Tree Protocol, which provides for faster spanning tree convergence after a topology change - Multiple Spanning Tree (MSTP) 802.1s: Multiple Spanning Tree Protocol. The MSTP protocol provides for multiple spanning tree instances |
| VLAN | - 802.1Q tag-based VLAN: Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs) - Port-based VLAN: A port member of a VLAN can be isolated to other isolated ports on the same VLAN and Private VLAN - Private VLAN Edge (PVE): Private VLANs are based on the source port mask, and there are no connections to VLANs. This means that VLAN IDs and Private VLAN IDs can be identical - Voice VLAN: The Voice VLAN feature enables voice traffic forwarding on the Voice VLAN - Guest VLAN: The IEEE 802.1X Guest VLAN feature allows a guest VLAN to be configured for each 802.1X port on the device to provide limited services to non-802.1X-compliant clients - Q-in-Q (double tag) VLAN: Business customers of service providers often have specific requirements for VLAN IDs and the number of VLANs to be supported - 802.1v Protocol VLAN: Classifying multiple protocols into a single VLAN often imposes VLAN boundaries that are inappropriate for some of the protocols, requiring the presence of a non-standard entity to relay between VLANs the frames bearing the protocols for which the VLAN boundaries are inappropriate - MAC-based VLAN: The MAC-based VLAN feature allows incoming untagged packets to be assigned to a VLAN and thus classify traffic based on the source MAC address of the packet - IP Subnet-Based VLAN: In an IP subnet–based VLAN, all the end workstations in an IP subnet are assigned to the same VLAN. In this VLAN, users can move their workstations without reconfiguring their network addresses - Management VLAN: Management VLAN is used for managing the switch from a remote location by using protocols such as telnet, SSH, SNMP, syslog etc |
| LACP Trunking | Link Aggregation Control Protocol (LACP) IEEE 802.3ad: Controls whether LACP is enabled on this switch port. LACP will form an aggregation when 2 or more ports are connected to the same partner - Up to 26 groups - Up to 16 ports per group |
| GARP VLAN Registration Protocol (GVRP/GVRP) | GVRP stands for GARP (Generic Attribute Registration Protocol) VLAN Registration Protocol. It’s a Layer 2 network protocol, for automatic configuration of switches in a VLAN network |
| DHCP Relay | - Relay of DHCP traffic to DHCP server in different VLAN - Works with DHCP Option 82 |
| IGMP v1/v2/v3 Snooping | IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups |
| IGMP Querier | IGMP querier is used to support a Layer 2 multicast domain of snooping switches in the absence of a multicast router |
| IGMP Proxy | IGMP snooping with proxy reporting or report suppression actively filters IGMP packets in order to reduce load on the multicast router |
| MLD v1/v2 Snooping | Delivers IPv6 multicast packets only to the required receivers |
| Multicast VLAN Registration (MVR) | It uses a dedicated manually configured VLAN, called the multicast VLAN, to forward multicast traffic over Layer 2 network in conjunction with IGMP snooping |
| Layer 3 Switching |  |
| IPv4 Static Routing | IPv4 Unicast: Static routing |
| IPv6 Static Routing | IPv6 Unicast: Static routing |
| Quality of Service |  |
| Hardware Queue | Supports 8 hardware queues |
| Classification | - Port based: Traffic QoS by Port - 802.1p: VLAN priority based Layer 2 CoS QoS, Class of service is a parameter used in data and voice protocols to differentiate the types of payloads contained in the packet being transmitted - DSCP based Differentiated Services (DiffServ) Layer 3 DSCP QoS: IP packets can carry either an IP precedence (IPP) value or a Differentiated Services Code Point (DSCP) value. QoS supports the use of either value because DSCP values are backward-compatible with IP precedence values - Classification and re-marking TCP/IP ACLs: QoS by ACL |
| Rate Limiting | - Ingress policer - Egress shaping and rate control - Per port |
| Scheduling | Strict priority and weighted round-robin (WRR): Weighted Round Robin is a scheduling algorithm that uses weights assigned to queues to determine how much data will be emptied from a queue before moving to the next queue |
| Security |  |
| ACLs | Supports up to 512 entries. Drop or rate limitation based on: - Source and destination MAC, VLAN ID or IP address, protocol, port - Differentiated services code point (DSCP) / IP precedence - TCP/ UDP source and destination ports - 802.1p priority - Ethernet type - Internet Control Message Protocol (ICMP) packets - TCP flag |
| Port Security | Locks MAC addresses to ports, and limits the number of learned MAC address |
| IP Source Guard | Prevents illegal IP address from accessing to specific port in the switch |
| Storm Control | Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port |
| IEEE 802.1X | - IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions - Supports IGMP-RADIUS based 802.1X - Dynamic VLAN assignment |
| TACACS+ | Supports TACACS+ authentication. Switch as a client |
| Secure Shell (SSH) | SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported |
| Secure Sockets Layer (SSL) | SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch |
| HTTPs and SSL (Secured Web) | Hyper Text Transfer Protocol Secure (HTTPS) is the secure version of HTTP |
| BPDU Guard | The BPDU guard, an enhancement to STP, removes a node that reflects BPDUs back in the network. It enforces the STP domain borders and keeps the active topology predictable by not allowing any network devices behind a BPDU guard-enabled port to participate in STP |
| DHCP Snooping | A feature acts as a firewall between untrusted hosts and trusted DHCP servers |
| Loop Protection | To prevent unknown unicast, broadcast and multicast loops in Layer 2 switching configurations. |
| Management |  |
| DHCP | - DHCP Server: Support DHCP server to assign IP to DHCP clients - DHCP client: The Dynamic Host Configuration Protocol (DHCP) is a standardized network protocol used on Internet Protocol (IP) networks for dynamically distributing network configuration parameters, such as IP addresses for interfaces and services |
| Event/Error Log | Support SNMP Trap/Syslog/SMTP |
| SNMP | SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM) |
| Remote Monitoring (RMON) | Embedded RMON agent supports RMON groups 1,2,3,9 (history, statistics, alarms, and events) for enhanced traffic management, monitoring and analysis |
| Firmware Upgrade | - Web browser upgrade (HTTP/ HTTPs) and TFTP - Upgrade through console port as well |
| Configuration Export/Import | update of the firmware controlling the switch |
| Port Mirroring | Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported |
| IEEE 802.1ab (LLDP) | - Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network - Support LLDP-MED (ANSI/TIA-1057) extensions |
| UPnP | The Universal Plug and Play Forum, an industry group of companies working to enable device-to-device interoperability by promoting Universal Plug and Play |
| CDP Aware | The CDP operation is restricted to decoding incoming CDP frames (The switch doesn’t transmit CDP frames). CDP frames are only decoded if LLDP on the port is enabled |
| s-Flow | The industry standard for monitoring high speed switched networks. It gives complete visibility into the use of networks enabling performance optimization, accounting/billing for usage, and defense against security threats |
| Web GUI Interface | Built-in switch configuration utility for browser-based device configuration |
| CLI | For users to configure/manage switches in command line modes |
| Dual Image | Independent primary and secondary images for backup while upgrading |
| NTP | Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched |
| Switch Management | - HTTP/HTTPs - SSH - DHCP Client/ DHCPv6 Client - Telnet Client - IPv6 Management |
| Diagnostics | - Cable diagnostics - Ping - Syslog |
| Device Management System (DMS) |  |
| Graphical Monitoring | - Topology view: Support intuitive way to configure and manage switches and devices with visual relations - Floor view: It’s easy to drag and drop PoE devices and help you to build smart workforces - Map view: Enhance efficiency to drag and drop devices and monitor surroundings on google map - Display visual chart of network traffic of all devices and monitor every port at any time from switches |
| Find my Switch | Search your real switches quickly and manage directly |
| Technical Support Online | Provide technical assistance to the user remotely |
| Traffic Monitoring | Display visual chart of network traffic of all devices and monitor every port at any time from switches |
| Trouble Shooting | - Network diagnostic between master switch and devices - Support protection mechanism, such as rate-limiting to protect your devices from brute-force downloading |
| Power over Ethernet (PoE) |  |
| Port Configuration | Supports per port PoE configuration function |
| PoE Scheduling | Supports per port PoE scheduling to turn on/off the PoE devices (PDs). |
| Auto-checking | Check the link status of PDs. Reboot PDs if there is no responses |
| Power Delay | The switch provides power to the PDs based on delay time when PoE switch boots up, in order to protect switch from misuse of the PDs |