PSXS-3810ATS

Multi-gigabit 10G PoE++ Switch

Overview

PSXS-3810ATS switch is next-generation managed (100M/1G/2.5G/5G/10G) Multi-gigabit PoE++ switch offering powerful L2 and basic L3 features with better functionality and usability. In addition to the extensive management features.

PSXS-3810ATS delivers 8 (100M/1G/2.5G/5G/10G Multi-Gig) RJ45 and 2 (1G/10G) SFP+ ports and RJ45 Console port. PSXS-3810ATS provides high HW performance for aggregation/access network applications.

PSXS-3810ATS supports 8 PoE++ (PoE output max. 90W per port). This switch provides high HW performance and environment flexibility for SMBs and Enterprises.

Key Features

- Fully L2 features provide easier manageability, security and QoS

- IEEE 1588v2 PTP

- ITU-T G.8031 Ethernet Linear Protection Switching (EPS)

- ITU-T G.8032 Ethernet Ring Protection Switching (ERPS)

- DHCP Server

- IPv4/IPv6 L3 static route

- RIP v1/v2 L3 dynamic routing

- OSPFv2/v3 L3 dynamic routing

- PoE Port configuration and scheduling

- 802.3af/802.3at/802.3bt high power PoE++ standard

Benefits

- Feature-rich Ethernet Switch for High Bandwidth Aggregation Applications

The switch delivers advanced functionality in this Multi-Gig Ethernet switch including L2 and L3 switch features, etc. It also enhanced security features such as IP source guard, Access Control List guard your network from unwanted or unauthorized access.

It helps users to build on the market-leading price/performance with Multi-Gig Ethernet switch, and provide secure, reliable for aggregation network deployments.

- Exceptional Precision with IEEE 1588v2

The switch performs IEEE1588v2 PTP function, which is implemented in the hardware architecture to assure low latency and high time accuracy – which is critical for delay-sensitive financial and mobile applications.

- ITU-T G.8032 (ERPS)

Ethernet Ring Protection Switching (ERPS) is an effort at ITU-T under G.8032 Recommendation to provide sub-50ms protection and fast switching recovery for Ethernet traffic in a ring topology, and at the same time ensuring that there are no loops formed at the Ethernet layer.

These features contribute to significant reduction in operational expenditures and allows for troubleshooting without expensive truck rolls.

Specifications

Port Configuration

| Total Ports | RJ45 (100M/1G/2.5G/5G/10G) | SFP+ (1G/10G) | SFP28 (1G/10G/25G) | Console |
| --- | --- | --- | --- | --- |
| 10 | 8 | 2 | -- | RJ45 |

Hardware Performance

| Forwarding Capacity | Switching Capacity | Mac Table | Jumbo Frames |
| --- | --- | --- | --- |
| 148.809Mbps | 200Gbps | 32K | 10K Bytes |

Environmental Range

| Operating Temperature | Storage Temperature | Altitude |
| --- | --- | --- |
| 0°C to 40°C | -20 to 70°C | < 3000m |

Dimension, Weights, Humidity

| Dimension (WxHxD) | Weight | Operating Humidity |
| --- | --- | --- |
| Millimeter | Kilograms |  |
| 442x44x260 | TBD | 10% to 90%, non-condensing |

Power Supply / Voltage and Frequency

Power Input Voltage and Frequency

AC Input: 100-240 VAC, 50~60 Hz

PoE Power Capacity

| Available PoE Power | Number of Ports That Support PoE(80.3af 15.4W), PoE+(802.3at 30.0W), PoE++ (802.3bt Type-3 60W & Type-4 90W) |
| --- | --- |
| 720W | Each of port 1 - 8 support PoE/ PoE+/PoE++ within available PoE Power |

Compliance

EMC and Safety

- SHALL comply with European Directives for CE marking (LVD with EN 62368-1) - SHALL comply with FCC Part 15 Class A - Other by Request

Software Features

| Layer 2 Switching |  |  |
| --- | --- | --- |
| Spanning Tree Protocol (STP) |  | - Standard Spanning Tree 802.1d - Rapid Spanning Tree (RSTP) 802.1w - Multiple Spanning Tree (MSTP) 802.1s |
| Trunking |  | Link Aggregation Control Protocol (LACP) IEEE 802.3ad |
| VLAN |  | Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs) - Port-based VLAN - 802.1Q tag-based VLAN - MAC-based VLAN - Management VLAN - Private VLAN Edge (PVE) - Q-in-Q (double tag) VLAN - Voice VLAN - GARP VLAN Registration Protocol (GVRP) |
| DHCP Relay |  | - Relay of DHCP traffic to DHCP server in different VLAN. - Works with DHCP Option 82 |
| IGMP v1/v2/v3 Snooping |  | IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups |
| IGMP Querier |  | IGMP querier is used to support a Layer 2 multicast domain of snooping switches in the absence of a multicast router |
| IGMP Proxy |  | IGMP snooping with proxy reporting or report suppression actively filters IGMP packets in order to reduce load on the multicast router |
| MLD v1/v2 Snooping |  | Delivers IPv6 multicast packets only to the required receivers |
| Layer 3 Switching |  |  |
| IPv4 Static routing |  | IPv4 Unicast: Static routing |
| IPv6 Static routing |  | IPv6 Unicast: Static routing |
| RIP v1/v2 |  | Routing Information Protocol (RIP) protocol are the intra-domain (interior) routing protocol which is based on distance vector routing and it is used inside an autonomous system. |
| OSPF v2/v3 |  | OSPF is a link-state routing protocol. It is designed to be run internal to a single Autonomous System. Each OSPF router maintains an identical database describing the Autonomous System's topology. From this database, a routing table is calculated by constructing a shortest-path tree. |
| Security |  |  |
| Secure Shell (SSH) |  | SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported |
| Secure Sockets Layer (SSL) |  | SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch |
| IEEE 802.1X |  | - IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions - Supports IGMP-RADIUS based 802.1X - Dynamic VLAN assignment |
| Layer 2 Isolation Private VLAN Edge |  | PVE (also known as protected ports) provides L2 isolation between clients in the same VLAN. Supports multiple uplinks |
| Port Security |  | Locks MAC addresses to ports, and limits the number of learned MAC address |
| IP Source Guard |  | Prevents illegal IP address from accessing to specific port in the switch |
| RADIUS/ TACACS+ |  | Supports RADIUS and TACACS+ authentication. Switch as a client |
| Storm Control |  | Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port |
| DHCP Snooping |  | A feature acts as a firewall between untrusted hosts and trusted DHCP servers |
| ACLs |  | Supports up to 256 entries. Drop or rate limitation based on: - Source and destination MAC, VLAN ID or IP address, protocol, port, - Differentiated services code point (DSCP) / IP precedence - TCP/ UDP source and destination ports - 802.1p priority - Ethernet type - Internet Control Message Protocol (ICMP) packets - TCP flag |
| Quality of Service |  |  |
| Hardware Queue |  | Supports 8 hardware queues |
| Scheduling |  | - Strict priority and weighted round-robin (WRR) - Queue assignment based on DSCP and class of service |
| Classification |  | - Port based - 802.1p VLAN priority based - IPv4/IPv6 precedence / DSCP based - Differentiated Services (DiffServ) - Classification and re-marking ACLs |
| Rate Limiting |  | - Ingress policer - Egress shaping and rate control - Per port |
| Management |  |  |
| DHCP Server |  | Support DHCP server to assign IP to DHCP clients |
| Remote Monitoring (RMON) |  | Embedded RMON agent supports RMON groups 1,2,3,9 (history, statistics, alarms, and events) for enhanced traffic management, monitoring and analysis |
| Port Mirroring |  | Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported. |
| UPnP |  | The Universal Plug and Play Forum, an industry group of companies working to enable device-to-device interoperability by promoting Universal Plug and Play |
| s-Flow |  | The industry standard for monitoring high speed switched networks. It gives complete visibility into the use of networks enabling performance optimization, accounting/billing for usage, and defense against security threats |
| IEEE 802.1ab (LLDP) |  | - Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network - Support LLDP-MED extensions |
| Web GUI Interface |  | Built-in switch configuration utility for browser-based device configuration |
| CLI |  | For users to configure/manage switches in command line modes |
| Dual Image |  | Independent primary and secondary images for backup while upgrading |
| SNMP |  | SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM) |
| Firmware Upgrade |  | - Web browser upgrade (HTTP/ HTTPs) and TFTP - Upgrade through console port as well |
| Other Management |  | - HTTP/HTTPs; SSH - DHCP Client/ DHCPv6 Client - Cable Diagnostics - Ping - Syslog - Telnet - IPv6 Management |
| NTP |  | Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched |
| HW Monitoring |  | Temperature Detection and Alarm |
| HW Watchdog |  | Supported to resume operation from CPU hang up |
| Loop Protection |  |  |
| ITU-T G.8031 |  | Supports ITU-T G.8031 Ethernet Linear Protection Switching |
| ITU-T G.8032 |  | Supports ITU-T G.8032 Ethernet Ring Protection Switching |
| Loop Detection |  | Supports Loop Detection and Protection |
| Power over Ethernet (PoE) |  |  |
| Port Configuration | Supports per port PoE configuration function |  |
| PoE Scheduling | Supports per port PoE scheduling to turn on/off the PoE devices (PDs). |  |
| Auto-checking | Check the link status of PDs. Reboot PDs if there is no responses |  |
| Power Delay | The switch provides power to the PDs based on delay time when PoE switch boots up, in order to protect switch from misuse of the PDs |  |

| Clock function |  |
| --- | --- |
| Sync-E | Support for ZL30772 and ZLS30380 advanced servo integration |
| IEEE 1588v2 PTP | Support IEEE 1588 v2 PTP (Precision Time Protocol) |
| G.8265.1/ G.8275.1/G.8275.2 | Support G.8265.1/8575.1/8275.2 profile Timing characteristics of Telecom Boundary Clocks, Class B |

[Remark]: All software features listed is subject to change based on MCHP Turnkey 2020.12 capability.