PSGS-5508F Unmanaged GbE PoE Switch

Overview

GS-5308 Unmanaged Switch is a plug-and-play Ethernet Switch offering easy way to make the transition to Gigabit Ethernet and increase the speed of your network connection. The energy efficient, built to last, and rigorously tested provide the reliability businesses need.

GS-5308 delivers 8 (10M/100M/1G) RJ45 ports. GS-5308 provides high HW performance and environment flexibility for SMBs and Enterprises.

GS-5308 is ideal to provide entry-level networking for Enterprise Applications which demands GbE speed. The 802.1p traffic prioritization and jumbo frame support allow for seamless integration with more sophisticated networks.

Key Features

- GbE Interfaces

- Green/Power Saving

Benefits

- GbE Interfaces

All interfaces including copper or fiber can deliver GbE bandwidth for backbone applications

- Green/Power Saving

Energy-saving capabilities save your money by lower power consumption

Specifications

Port Configuration

| Total Ports | RJ45 (10M/100M/1G) | Uplinks (100M/1G) | Console |
| --- | --- | --- | --- |
| 8 | 8 | -- | -- |

Hardware Performance

| Forwarding Capacity | Switching Capacity | Mac Table | Jumbo Frames |
| --- | --- | --- | --- |
| 11.9 Mpps | 16 Gbps | 8 K | 9216 Bytes |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Altitude |  |
| --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | Feet | Meters |
| 32 to 122 | 0 to 50 | -4 to 158 | -20 to 70 | < 10000 | <3000 |

Dimension, Weights, Humidity

| Dimension (WxHxD) |  |  | Weight |  |  | Operating Humidity |
| --- | --- | --- | --- | --- | --- | --- |
| Millimeter | Inches |  | Kilograms | Pounds |  |  |
| 197 x 35.5 x 120 | 7.76 x 1.4 x 4.72 | 0.8 |  |  | 1.76 | 10% to 90% non-condensing |

Voltage and Frequency

| AC Input Voltage and Frequency |  |
| --- | --- |
| Voltage | 100-240 VAC |
| Frequency | 50~60 Hz |

Certification

Electromagnetic Emissions (EMC)

CE, FCC Part 15 Class A

Software Features

| Quality of Service |  |
| --- | --- |
| Classification | - Port based - 802.1p VLAN priority based |
| Rate Limiting | - Ingress policer - Egress shaping and rate control |