PSGS-5508L Unmanaged GbE PoE Switch

Overview

PSGS-5508L Unmanaged PoE+ Switch is a plug-and-play Ethernet Switch offering easy way to make the transition to Gigabit Ethernet and increase the speed of your network connection. The energy efficient, built to last, and rigorously tested provide the reliability businesses need.

PSGS-5508L delivers 8 (10M/100M/1G) RJ45/PoE+ (Support 802.3at/af, and total up to 65W) ports. PSGS-5508L provides high HW performance and environment flexibility for SMBs and Enterprises.

PSGS-5508L is ideal to provide entry-level networking for Enterprise Applications which demands GbE speed. The 802.1p traffic prioritization and jumbo frame support allow for seamless integration with more sophisticated networks.

Key Features

- GbE Interfaces

- Green/Power Saving

Benefits

- GbE Interfaces

All interfaces including copper or fiber can deliver GbE bandwidth for backbone applications

- Green/Power Saving

Energy-saving capabilities save your money by lower power consumption

Specifications

Port Configuration

| Total Ports | RJ45 (10M/100M/1G) | Uplinks (100M/1G) | Console |
| --- | --- | --- | --- |
| 8 | 8 | -- | -- |

Hardware Performance

| Forwarding Capacity | Switching Capacity | Mac Table | Jumbo Frames |
| --- | --- | --- | --- |
| 11.9 Mpps | 16 Gbps | 8 K | 9216 Bytes |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Altitude |  |
| --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | Feet | Meters |
| 32 to 122 | 0 to 50 | -4 to 158 | -20 to 70 | < 10000 | <3000 |

Dimension, Weights, Humidity

| Dimension (WxHxD) |  |  | Weight |  |  | Operating Humidity |
| --- | --- | --- | --- | --- | --- | --- |
| Millimeter | Inches |  | Kilograms | Pounds |  |  |
| 220 x 44 x 134 | 8.6 x 1.7 x 5.3 | 0.8 |  |  | 1.76 | 10% to 90% non-condensing |

Voltage and Frequency

| AC Input Voltage and Frequency |  |
| --- | --- |
| Voltage | 100-240 VAC |
| Frequency | 50~60 Hz |

PoE Power Capacity

| Available PoE Power | Number of Ports That Support PoE(15.4W) and PoE+(30.0W) |
| --- | --- |
| 65W | Each of port 1 - 8 support PoE/ PoE+ within available PoE Power |

Certification

Electromagnetic Emissions (EMC)

CE, FCC Part 15 Class A

Software Features

| Quality of Service |  |
| --- | --- |
| Classification | - Port based - 802.1p VLAN priority based |
| Rate Limiting | - Ingress policer - Egress shaping and rate control |