
PSGS-2626K L2 Managed GbE PoE Switch
[image: image1.jpg]PSGS-2326KF

Power LED Port Status LEDs
Console Port

Reset Button 10/100/1000 RJ45 Ports 100/1000 RJ45/SFP  100/1000
Combo Ports SFP+ Ports




Overview
PSGS-2626K L2 Managed PoE Switch features complete L2 switching capabilities for an easy-to-administer solution and provides the best combination of features, performance, and value for Enterprise networks via fiber or copper connections. 

PSGS-2626K delivers 20 (10M/100M/1G) RJ45/PoE+ (Support 802.3at/af, and total up to 370W) ports, 4 Combo GbE RJ45/SFP ports, 2 GbE SFP ports and RJ45 Console port. PSGS-2626K provides high HW performance and environment flexibility for SMBs and Enterprises.

PSGS-2626K is ideal for environments that require advanced features for granular control which is a must for easy network configuration and management where you need for a successful installation of IP Phone, IP Cam, Wifi-AP or any other Enterprise Applications.

Key Features

· L2 Managed features provide easier manageability, robust security and QoS
· Easy-Port-Configuration in the IP Phone, IP Camera or Wireless environment 
· PoE Port configuration and scheduling

· 802.3at high power PoE plus standard

· IEEE 802.3az EEE Energy Efficient Ethernet standard for green Ethernet
Benefits 

· Feature-rich Ethernet Switch for Enterprise-class

The switch delivers advanced functionality in L2+ managed switch including Layer 3 static route, DHCP server, IPv6 support, LLDP, etc. It also has comprehensive security features such as IP source guard and ACL to guard your network from unauthorized access.

It helps users to build on the market-leading price/performance with L2+ Managed GbE PoE switch, and provide secure, reliable and ease of use for enterprise/SMB deployments.

· Easy to Install, Configure and Troubleshoot by Device Management System

The DMS provides embedded functions to facilitate devices management at anytime and anywhere. Its user-friendly interface helps users to manage devices intuitively. 

It supports various IP device types (e.g. IP-phone, IP-camera, WiFi-AP) for end users to enhance manageability and save time/cost during installation/maintenance stages.
· Lower Total Cost of Ownership (TCO) with Energy-efficient Design

It is designed to help customers to reduce power consumption and lower the TCO by Energy Efficient Ethernet (IEEE 802.3az) features. It can be used for customers to build a green Ethernet networking environment.

· Advanced Power over Ethernet Management

The model includes PoE+ options to power IP devices with power-saving features like Power scheduling and PoE configuration.

Specifications
Port Configuration
	Total Ports
	RJ45 (10M/100M/1G)
	Uplinks (100M/1G)
	Console

	26
	20
	2 SFP
4 RJ45/SFP combo
	RJ45


Hardware Performance

	Forwarding Capacity 
	Switching Capacity 
	Mac Table
	Jumbo Frames

	38.7 Mpps
	52 Gbps
	8 K
	9216 Bytes


Environmental Range

	Operating Temperature
	Storage Temperature
	Altitude

	Fahrenheit
	Centigrade
	Fahrenheit
	Centigrade
	Feet
	Meters

	32 to 122
	0 to 50
	-4 to 158
	-20 to 70
	< 10000
	< 3000 


Dimension, Weights, Humidity
	Dimension (WxHxD)
	Weight
	Operating Humidity

	Millimeter
	Inches
	Kilograms
	Pounds
	

	442 x 44 x 300
	17.4 x 1.73 x 11.8
	3.8
	8.4
	10% to 90%  non-condensing


Voltage and Frequency
	AC Input Voltage and Frequency

	Voltage 
	100-240 VAC  

	Frequency
	50~60 Hz


PoE Power Capacity

	Available PoE Power
	Number of Ports That Support PoE(15.4W) and PoE+(30.0W)

	370W
	Each of port 1 - 24 support PoE/ PoE+ within available PoE Power


Certification

	Electromagnetic Emissions (EMC) 

	CE, FCC Part 15 Class A


Software Features 
	Layer 2 Switching 

	Spanning Tree Protocol (STP)
	· Standard Spanning Tree 802.1d 

· Rapid Spanning Tree (RSTP) 802.1w 

· Multiple Spanning Tree (MSTP) 802.1s

	Trunking
	Link Aggregation Control Protocol (LACP) IEEE 802.3ad

· Up to 13 groups 

· Up to 16 ports per group

	VLAN
	Supports up to 4K VLANs simultaneously (out of 4096 VLAN IDs)

· Port-based VLAN

· 802.1Q tag-based VLAN

· MAC-based VLAN

· Management VLAN

· Private VLAN Edge (PVE)

· Q-in-Q (double tag) VLAN

· Voice VLAN

· GARP VLAN Registration Protocol (GVRP)

	DHCP Relay 
	· Relay of DHCP traffic to DHCP server in different VLAN. 

· Works with DHCP Option 82

	IGMP v1/v2/v3 Snooping
	IGMP limits bandwidth-intensive multicast traffic to only the requesters. Supports 1024 multicast groups 

	IGMP Querier
	IGMP querier is used to support a Layer 2 multicast domain of snooping switches in the absence of a multicast router

	IGMP Proxy
	IGMP snooping with proxy reporting or report suppression actively filters IGMP packets in order to reduce load on the multicast router

	MLD v1/v2 Snooping
	Delivers IPv6 multicast packets only to the required receivers

	Device Management System (DMS)

	Graphical 

Monitoring
	· Topology view: Support intuitive way to configure and manage switches and devices with visual relations
· Floor view: It’s easy to drag and drop PoE devices and help you to build smart workforces
· Map view: Enhance efficiency to drag and drop devices and monitor surroundings on google map

	Find my Switch
	Search your real switches quickly and manage directly.

	Traffic Monitoring
	Display visual chart of network traffic of all devices and monitor every port at any time from switches

	Trouble Shooting
	· Network diagnostic between master switch and devices
· Support protection mechanism, such as rate-limiting to protect your devices from brute-force downloading


	Layer 3 Switching 

	IPv4 Static Routing
	IPv4 Unicast: Static routing

	IPv6 Static Routing
	IPv6 Unicast: Static routing

	Security  

	Secure Shell (SSH)
	SSH secures Telnet traffic in or out of the switch, SSH v1 and v2 are supported

	Secure Sockets Layer (SSL)
	SSL encrypts the http traffic, allowing advanced secure access to the browser-based management GUI in the switch

	IEEE 802.1X
	· IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions

· Supports IGMP-RADIUS based 802.1X

· Dynamic VLAN assignment

	Layer 2 Isolation Private VLAN Edge 
	PVE (also known as protected ports) provides L2 isolation between clients in the same VLAN. Supports multiple uplinks

	Port Security
	Locks MAC addresses to ports, and limits the number of learned MAC address

	IP Source Guard
	Prevents illegal IP address from accessing to specific port in the switch

	RADIUS/ TACACS+ 
	Supports RADIUS and TACACS+ authentication. Switch as a client

	Storm Control
	Prevents traffic on a LAN from being disrupted by a broadcast, multicast, or unicast storm on a port

	DHCP Snooping
	A feature acts as a firewall between untrusted hosts and trusted DHCP servers

	ACLs
	Supports up to 256 entries. Drop or rate limitation based on:

· Source and destination MAC, VLAN ID or IP address, protocol, port,

· Differentiated services code point (DSCP) / IP precedence

· TCP/ UDP source and destination ports

· 802.1p priority

· Ethernet type

· Internet Control Message Protocol (ICMP) packets

· TCP flag

	Quality of Service 

	Hardware Queue 
	Supports 8 hardware queues

	Scheduling
	· Strict priority and weighted round-robin (WRR)

· Queue assignment based on DSCP and class of service

	Classification
	· Port based

· 802.1p VLAN priority based

· IPv4/IPv6 precedence / DSCP based

· Differentiated Services (DiffServ)

· Classification and re-marking ACLs

	Rate Limiting
	· Ingress policer

· Egress shaping and rate control

· Per port

	Management

	DHCP Server
	Support DHCP server to assign IP to DHCP clients

	Remote Monitoring (RMON)
	Embedded RMON agent supports RMON groups 1,2,3,9 (history, statistics, alarms, and events) for enhanced traffic management, monitoring and analysis

	Port Mirroring
	Traffic on a port can be mirrored to another port for analysis with a network analyzer or RMON probe. Up to N-1 (N is Switch’s Ports) ports can be mirrored to single destination port. A single session is supported.  

	UPnP
	The Universal Plug and Play Forum, an industry group of companies working to enable device-to-device interoperability by promoting Universal Plug and Play 

	s-Flow
	The industry standard for monitoring high speed switched networks. It gives complete visibility into the use of networks enabling performance optimization, accounting/billing for usage, and defense against security threats 

	IEEE 802.1ab (LLDP) 
	· Used by network devices for advertising their identities, capabilities, and neighbors on an IEEE 802ab local area network

· Support LLDP-MED extensions

	Web GUI Interface
	Built-in switch configuration utility for browser-based device configuration 

	CLI
	For users to configure/manage switches in command line modes

	Dual Image
	Independent primary and secondary images for backup while upgrading 

	SNMP 
	SNMP version1, 2c and 3 with support for traps, and SNMP version 3 user-based security model (USM)

	Firmware Upgrade
	· Web browser upgrade (HTTP/ HTTPs) and TFTP

· Upgrade through console port as well

	NTP
	Network Time Protocol (NTP) is a networking protocol for clock synchronization between computer systems over packet-switched

	Other Management
	· HTTP/HTTPs; SSH

· DHCP Client/ DHCPv6 Client 

· Cable Diagnostics

· Ping

· Syslog

· Telnet 
· IPv6 Management

	Power over Ethernet (PoE)

	Port Configuration
	Supports per port PoE configuration function

	PoE Scheduling
	Supports per port PoE scheduling to turn on/off the PoE devices (PDs).

	Auto-checking
	Check the link status of PDs. Reboot PDs if there is no responses.

	Power Delay
	The switch provides power to the PDs based on delay time when PoE switch boots up, in order to protect switch from misuse of the PDs


PAGE  

- 6 -
PSGS-2626K Datasheet 


