PSGS-2618G

Quick Installation Guide

2016, _________ Corporation. All rights reserved. All brand and product names are trademarks or registered trademarks of their respective companies

ABOUT THIS GUIDE

PURPOSE
This guide gives specific information on how to operate and use the management functions of the switch.

AUDIENCE 
The guide is intended for use by network administrators who are responsible for operating and maintaining network equipment; consequently, it assumes a basic working knowledge of general switch functions, the Internet Protocol (IP), and Simple Network Management Protocol (SNMP).

CONVENTIONS
The following conventions are used throughout this guide to show information:

WARRANTY

See the Customer Support/ Warranty booklet included with the product.

A copy of the specific warranty terms applicable to your Manufacture products and replacement parts can be obtained from your Manufacture Sales and Service Office authorized dealer.

|  | NOTE: Emphasizes important information or calls your attention to related features or instructions. |
| --- | --- |
|  | WARNING: Alerts you to a potential hazard that could cause personal injury. |
|  | CAUTION: Alerts you to a potential hazard that could cause loss of data, or damage the system or equipment. |

COMPLIANCES AND SAFETY STATEMENTS

FCC-CLASS A

This equipment has been tested and found to comply with the limits for a Class A computing device pursuant to Subpart J of part 15 of FCC Rules, which are designed to provide reasonable protection against such interference when operated in a commercial environment.

This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

You are cautioned that changes or modifications not expressly approved by the party responsible for compliance could void your authority to operate the equipment.

You may use unshielded twisted-pair (UTP) for RJ-45 connections - Category 3 or better for 10 Mbps connections, Category 5 or better for 100 Mbps connections, Category 5, 5e, or 6 for 1000 Mbps connections. For fiber optic connections, you may use 50/125 or 62.5/125 micron multimode fiber or 9/125 micron single-mode fiber.

CE MARK DECLARATION

OF CONFORMANCE FOR EMC AND SAFETY

This equipment has been tested and found to comply with the protection requirements of European Emission Standard EN55022/EN61000-3 and the Generic European Immunity Standard EN55024.

EMC:

| EN55022(2006)+A1:2007/CISPR 22:2006+A1:2006 | Class A 4K V CD, 8KV, AD |
| --- | --- |
| IEC61000-4-2 (2001) | 3V/m |
| IEC61000-4-3( 2002) | 1KV – (power line), 0.5KV – (signal line) |
| IEC61000-4-4(2004) | Line to Line: 1KV, Line to Earth: 2KV |
| IEC61000-4-5 (2001) | 130dBuV(3V) Level 2 |
| IEC61000-4-6 (2003) | 1A/m |
| IEC61000-4-8 (2001) | Voltage dips: >95%, 0.5period, 30%, 25periods |
| IEC61000-4-11(2001) | Voltage interruptions: >95%, 250periods |

|  | CAUTION: Circuit devices are sensitive to static electricity, which can damage their delicate electronics. Dry weather conditions or walking across a carpeted floor may cause you to acquire a static electrical charge. To protect your device, always: - Touch the metal chassis of your computer to ground the static electrical charge before you pick up the circuit device. - Pick up the device by holding it on the left and right edges only. - If you need using outdoor device connect to this device with cable then you need to addition an arrester on the cable between outdoor device and this device. Fig. Addition an arrester between outdoor device and this switch - The switch supports the SFP Vendor includes: Manufacture, Agilent and Avago. |
| --- | --- |

|  | NOTE: The switch is indoor device; if it will be used in outdoor environment or connects with some outdoor device, then it must use a lightning arrester to protect the switch |
| --- | --- |

|  | WARNING: - Self-demolition on Product is strictly prohibited. Damage caused by self-demolition will be charged for repairing fees. - Do not place product at outdoor or sandstorm. - Before installation, please make sure input power supply and product specifications are compatible to each other. - To reduce the risk of electric shock. Disconnect all AC or DC power cord and RPS cables to completely remove power from the unit. - Before importing / exporting configuration please make sure the firmware version is always the same. - After firmware upgrade, the switch will remove the configuration automatically to latest firmware version. |
| --- | --- |

RELATED PUBLICATIONS

The following publication gives specific information on how to operate and use the management functions of the switch:

The User’s Manual

REVISION HISTORY

This section summarizes the changes in each revision of this guide.

| Release | Date | Revision |
| --- | --- | --- |
| Initial Release | 05/12/2016 | A1 |

Contents

ABOUT THIS GUIDE	ii

COMPLIANCES AND SAFETY STATEMENTS	iii

INTRODUCTION	1

OVERVIEW	1

DESCRIPTION OF HARDWARE	4

NETWORK PLANNING	7

INTRODUCTION TO SWITCHING	7

APPLICATION EXAMPLES	7

INSTALLING THE SWITCH	10

SELECTING A SITE	10

ETHERNET CABLING	10

EQUIPMENT CHECKLIST	11

PACKAGE CONTENTS	11

MOUNTING	11

INSTALLING AN OPTIONAL SFP TRANSCEIVER	13

CONNECTING TO A POWER SOURCE	15

CONNECTING TO THE CONSOLE PORT	15

Operation of Web-based Management	18

MAKING NETWORK CONNECTIONS	21

CONNECTING NETWORK DEVICES	21

TWISTED-PAIR DEVICES	21

FIBER OPTIC SFP DEVICES	22

CABLE LABELING AND  CONNECTION RECORDS	25

TROUBLESHOOTING	26

Basic Troubleshooting Tips	26

POWER AND COOLING PROBLEMS	28

Installation	28

IN-BAND ACCESS	28

CABLES	29

TWISTED-PAIR CABLE AND PIN ASSIGNMENTS	29

STRAIGHT- THROUGH WIRING	30

1000BASE-T PIN ASSIGNMENTS	31

CABLE TESTING FOR EXISTING CATEGORY 5 CABLE	31

FIBER STANDARDS	32

SPECIFICATIONS	33

PHYSICAL CHARACTERISTICS	33

SWITCH FEATURES	34

MANAGEMENT FEATURES	34

STANDARDS	34

COMPLIANCES	35

COMPLIANCES	36

INTRODUCTION

OVERVIEW

PSGS-2618G Web Smart+ Managed PoE+ Switch is a next-generation Ethernet Switch offering powerful L2 features, Layer 3 Static Route, better PoE functionality and usability that delivers the cost-effectively business and transports Ethernet services via fiber or copper connections.

PSGS-2618G delivers 16 (10M/100M/1G) RJ45/PoE+ (Support 802.3at/af) ports and 2 Combo GbE RJ45/SFP ports. PSGS-2618G provides high HW performance and environment flexibility for SMBs and Enterprises.

PSGS-2618G is ideal to deliver management simplicity, optimum user experience, and lower cost. The embedded Device Managed System is designed to be extremely easy-to-use/manage/install IP Phone, IP Cam, or Wifi-AP for Enterprise Applications.

Front of the Switch

Rear of the Switch

SWITCH ARCHITECTURE

The switch performs a wire-speed, non-blocking switching fabric. This allows wire-speed transport of multiple packets at low latency on all ports simultaneously. The switch also features full-duplex capability on all ports, which effectively doubles the bandwidth of each connection.

This switch uses store-and-forward technology to ensure maximum data integrity. With this technology, the entire packet must be received into a buffer and checked for validity before being forwarded. This prevents errors from being propagated throughout the network.

NETWORK MANAGEMENT OPTIONS

The switch can also be managed over the network with a web browser or Telnet application. The switch includes a built-in network management agent that allows it to be managed in-band using SNMP or RMON (Groups 1, 2, 3, 9) protocols. It also has an RJ45 console port connector on the front panel for out-of-band management. A PC may be connected to this port for configuration and monitoring out-of-band via a null-modem serial cable. (See Appendix B for wiring options.)

|  | NOTE: For a detailed description of the management features, refer to the User’s manual. |
| --- | --- |

DESCRIPTION OF HARDWARE

1000BASE-T PORTS

The switch contains 18 1000BASE-T RJ-45 ports. All RJ-45 ports support automatic MDI/MDI-X operation, auto-negotiation and IEEE 802.3x auto-negotiation of flow control, so the optimum data rate and transmission can be selected automatically.

SFP TRANSCEIVER SLOTS

The following table shows a list of transceiver types which have been tested with the switch. For an updated list of vendors supplying these transceivers, contact your local dealer. For information on the recommended standards for fiber optic cabling, see “1000 Mbps Gigabit Ethernet Collision Domain”.

Supported SFP Transceivers

| Media Standard | Fiber Diameter (microns) | Wavelength (nm) | Maximum Distance* |
| --- | --- | --- | --- |
| 1000BASE-SX | 50/125 62.5/125 | 850 850 | 550 m 275 m |
| 1000BASE-LX/ LHX/ XD/ZX | 9/125 9/125 | 1310 1550 | 10,30 km 30,50 km |
| 1000BASE-LX Single Fiber | 9/125 | TX-1310/RX-1550 Tx-1550/RX-1310 | 10,20 km 10,20 km |
| 1000BASE-T | N/A | N/A | 100 m |
| 100-FX | 62.5/125 9/125 | 1310 1310 | 2 km 20,40,60 km |

|  | NOTE: * Maximum distance may vary for different SFP vendors |
| --- | --- |

PORT AND SYSTEM STATUS LEDS

The PSGS-2618G switch includes a display panel for system and port indications that simplify installation and network troubleshooting. The LEDs, which are located on left hand side of the front panel for easy viewing. Details are shown below and described in the following tables.

Port Status LEDs

| LED | Condition | Status |
| --- | --- | --- |
| TP (Link/Act/Speed) | Green/Blink | Lit Green when TP link on 1000Mbps Amber when TP link on 10/100Mbps |
| TP (PoE) | Green/Off | Lit Green when PoE link good |
| SFP (Link/Act/Speed) | Green/Blink | Lit Green when SFP link good Lit Green when SFP link on 1000Mbps. Amber when SFP link on 100Mbps |

System Status LED

| SYSTEM LED | Condition | Status |
| --- | --- | --- |
| System | Green OFF | Lit when power is coming up |

Table 3 Mode Status LED

| LED | Condition | Status |
| --- | --- | --- |
| Link/ACT/Speed | Green Yellow OFF | Green when the link on 1000Mbps Yellow when the link on 100Mbps Off when the link on 10Mbps |
| PoE | Green OFF | Lit Green shows all LED of each port are in PoE Mode |

POWER SUPPLY SOCKET

There are a power sockets on the rear panel of the switch. For normal power supply, the PSGS-2618G Switch has standard power socket for AC power cord

Mode/Reset Button

By pressing the Mode/Reset Button for certain period of time, users can perform the following tasks.

- Change Port Status LED Mode 	to read the port status correctly in the two different modes (Link/Act/Speed mode or PoE mode).

- Reset the Switch 	to reboot and get the switch back to the previous configuration settings saved.

- Restore the Switch to Factory Defaults       to restore the original factory default settings back to the switch.

Note:	According to the table below, users can easily judge which task is being performed by reading the LED behaviors while pressing the Mode/Reset button.   Once the LED behaviors are correctly displayed, users may just release the button.

Table 4 Mode/Reset Button Descriptions

| Task to be Performed | Time Period of Pressing Button | SYS LED Behavior | Port Status LED Behavior |
| --- | --- | --- | --- |
| Change LED Mode | 0 ~ 2 seconds | ON Green | LED status will be changed according the mode selected. |
| Reset the Switch | 2 ~ 7 seconds | Blinking Green | ALL LEDs Light OFF |
| Restore to Defaults | 7 ~ 12 seconds | Blinking Green | ALL LEDs Stay ON |

NETWORK PLANNING

INTRODUCTION TO SWITCHING

A network switch allows simultaneous transmission of multiple packets, and it can partition a network more efficiently than bridges or routers. Therefore the switch has been recognized as one of the most important devices for today’s networking technology.

When performance bottlenecks are caused by congestion at the network access point such as file server, the device can be connected directly to a switched port. And, by using full-duplex mode, the bandwidth of the dedicated segment can be doubled to maximize throughput.

When networks are based on repeater (hub) technology, the distance between end stations is limited by a maximum hop count. However, a switch can subdividing the network into smaller and more manageable segments, and linking them to the larger network than it turns the hop count back to zero and removes the limitation.

A switch can be easily configured in any Ethernet, Fast Ethernet, or Gigabit Ethernet network to significantly increase bandwidth while using conventional cabling and network cards.

APPLICATION EXAMPLES

The PSGS-2618G Switch implements 16 Gigabit Ethernet TP ports with auto MDIX and 2 Gigabit TP/SFP combo ports which supports comprehensive types of fiber connection, such as LC and BiDi-LC modules. It is not only designed to segment your network, but also to provide a wide range of options in setting up network connections. Some typical applications are described below.

The switch is suitable for the following applications.

- Remote site application is used in Enterprise or SMB

- Peer-to-peer application is used in two remote offices

- Office network

- High Performance Requirement environment

- Advance Security for network safety application

- Suitable for data/ voice and video conference application

INSTALLING THE SWITCH

SELECTING A SITE

The Switch can be mounted in a standard 19-inch equipment rack (Via Optional Rack mount Kit) or on a flat surface. Be sure to follow the guidelines below when choosing a location.

- The site should:

  - Be at the center of all the devices you want to link and near a power outlet.

  - Be able to maintain its temperature within 0 to 45°C (32 to 113 °F) and its humidity within 10% to 90%, non-condensing.

  - Be accessible for installing, cabling and maintaining the devices.

  - Allow the status LEDs to be clearly visible.

- Make sure the twisted-pair Ethernet cable is always routed away from power lines, radios, transmitters or any other electrical interference.

- Make sure that PSGS-2618G Switch is connected to a separate grounded power outlet that provides 100 to 240 VAC, 50 to 60 Hz.

ETHERNET CABLING

To ensure proper operation when installing the switch into a network, make sure that the current cables are suitable for 100BASE-TX or 1000BASE-T operation. Check the following criteria against the current installation of your network:

- Cable type: Unshielded twisted pair (UTP) or shielded twisted pair (STP) cable with RJ-45 connectors; Category 5 or Category 5e with maximum length of 100 meters is recommend 100BASE-TX, and Category 5e or 6 with maximum length of 100 meters is recommend for 1000BASE-T.

- Protection from radio frequency interference emissions.

- Electrical surge suppression.

- Separation of electrical wires and data based network wiring.

- Safe connections with no damaged cables, connectors or shields.

RJ-45 Connections

SFP Transceiver

EQUIPMENT CHECKLIST

After unpacking this switch, please check the contents to be sure you have received all the components. Then, before beginning the installation, be sure you have all other necessary installation equipment.

PACKAGE CONTENTS

- PSGS-2618G GbE Management Switch

- Four adhesive rubber feet

- Mounting Accessory (for 19” Rack Shelf, Optional)

- This Installation Guide

- User’s Manual CD

- AC Power Cord

- RS-232 to RJ-45 Console Cable

|  | NOTE: Please notify your sales representative immediately if any of the aforementioned items is missing or damaged. |
| --- | --- |

|  | WARNING: The mini-GBICs are Class 1 laser devices. Avoid direct eye exposure to the beam coming from the transmit port. |
| --- | --- |

MOUNTING

The switch can be mounted in a standard 19-inch equipment rack or on a desktop or shelf. Mounting instructions for each type of site as follow.

RACK MOUNTING

Before rack mounting the switch, please pay attention to the following factors:

- Temperature: Since the temperature within a rack assembly may be higher than the ambient room temperature, check that the rack-environment temperature is within the specified operating temperature range (0 to 45 °C).

- Mechanical Loading: Do not place any equipment on top of a rack-mounted unit.

- Circuit Overloading: Be sure that the supply circuit to the rack assembly is not overloaded.

- Grounding: Rack-mounted equipment should be properly grounded.

TO Rack-mount Devices:

Step1. Attach the brackets to the device using the screws provided in the Mounting Accessory.

Attaching the Brackets

Step2. Mount the device in the rack (Via Optional Rack-Mount kit), using four rack-mounting screws (not provided). Be sure to secure the lower rack-mounting screws first to prevent the brackets being bent by the weight of the switch.

Installing the switch in a Rack

Step3. If installing a single switch only, turn to “Connection to a Power Source” at the end of this chapter.

Step4. If installing multiple switches, mount them in the rack, one below the other, in any order.

DESKTOP OR SHELF MOUNTING:

Step1. Attach the four adhesive rubber feet to the bottom of the first switch.

Attaching the Adhesive Rubber Feet

Step2. Set the device on a flat surface near an AC power source, making sure there are at least two inches of space on all sides for proper air flow.

Step3. If installing a single switch only, go to “Connecting to a Power Source” at the end of this Chapter.

Step4. If installing multiple switches, attach four adhesive feet to each one. Place each device squarely on top of the one below, in any order.

INSTALLING AN OPTIONAL SFP TRANSCEIVER

You can install or remove a mini-GBIC SFP from a mini-GBIC slot without having to power off the switch. Use only Manufacture mini-GBIC.

|  | NOTE: - The mini-GBIC slots are shared with the two 10/ 100/ 1000Base-T RJ-45 ports. If a mini-GBIC is installed in a slot, the associated RJ-45 port is disabled and cannot be used - The mini-GBIC ports operate only at full duplex. Half duplex operation is not supported. - Ensure the network cable is NOT connected when you install or remove a mini-GBIC. |
| --- | --- |

|  | CAUTION: Use only supported genuine Manufacture mini-GBICs with your switch. Non-Manufacture mini-GBIC might have compatible issue, and their use may result in product malfunction. |
| --- | --- |

Inserting an SFP Transceiver into a Slot

The SFP slots support the following optional SFP transceivers:

| Model Name | Description |
| --- | --- |
| SFP.LC | 1000Base-SX GE SFP Fiber Module, LC Multi-Mode 850nm |
| SFP.LC.M2 | 1000Base-SX GE SFP Fiber Module, LC Multi-Mode 1310nm 2km |
| SFP.LC.S10 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 10km |
| SFP.LC.S30 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 30km |
| SFP.LC.S50 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 50km |
| SFP.L5.S50 | 1000Base-LX GE SFP Fiber Module, LC Single-Mode 50km |
| SFP.BL5.S10 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 10km, 1310nm |
| SFP.BL3.S10 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 10km, 1550nm |
| SFP.BL5.S20 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 20km, 1550nm |
| SFP.BL3.S20 | 1000Base-LX GE SFP Fiber Module, Bidi LC Single-Mode 20km, 1310nm |
| SFP.FLC | 100Base-FX FE SFP Fiber Module, LC Multi-Mode |
| SFP.FLC.S20 | 100Base-FX FE SFP Fiber Module, LC Single-Mode 20km |

|  | CAUTION: We recommend the SFP Transceiver from the following vendors: - Manufacture Corporation - Agilient Technologies - AVAGO Technologies - Finisar Corporation |
| --- | --- |

TO Install a SFP transceiver, do the following:

Step1. Consider network and cabling requirements to select an appropriate SFP transceiver type.

Step2. Insert the transceiver with the optical connector facing outward and the slot connector facing down. Note that SFP transceivers are keyed so they can only be installed in one orientation.

Step3. Slide the SFP transceiver into the slot until it clicks into place.

|  | NOTE: SFP transceivers are not provided in the switch package. |
| --- | --- |

CONNECTING TO A POWER SOURCE

You can plug or remove power cord from AC power socket, to switch the power on and off.

Inserting the Power Cord to AC Power Socket

Step1. Insert the power cable plug directly into the AC Socket located at the back of the switch.

Step2. Plug the other end of the cable into a grounded, 3-Pin, AC power source.

Step3. Check the front-panel LEDs as the device is powered on to be sure the SYSTEM LED is lit. If not, check that the power cable is correctly plugged in.

|  | WARNING: For International use, you may need to change the AC line cord. You must use a line cord set that has been approved for the socket type in your country. |
| --- | --- |

CONNECTING TO THE CONSOLE PORT

The RJ-45 serial port on the switch’s front panel is used to connect to the switch for out-of-band console configuration. The command-line-driven configuration program can be accessed from a terminal or a PC running a terminal emulation program. The pin assignments used to connect to the serial port are provided in the following table

Serial Port (RJ-45) Pin-Out

2 TXD

5 GND

3 RXD

WIRING MAP FOR SERIAL CABLE

Serial Cable Wiring

| Switch’s 8-Pin Serial Port | Null Modem | PC’s 9-Pin DTE Port |
| --- | --- | --- |
| 6 RXD (receive data) | ----------------- | 3 TXD (transmit data) |
| 3 TXD (transmit data) | ----------------- | 2 RXD (receive data) |
| 5 SGND (Signal ground) | ------------------- | 5 SGND (Signal ground) |

|  | NOTE: No other pins are used. |
| --- | --- |

Plug in the Console Port

The serial port’s configuration requirements are as follows:

- Default Baud rate—115,200 bps

- Character Size—8 Characters

- Parity—None

- Stop bit—One

- Data bits—8

- Flow control—none

- Operation of Web-based Management

The default values of the managed switch are listed in the table below:

| IP Address | 192.168.1.1 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.254 |
| Username | admin |
| Password |  |

After the managed switch has been finished configuration in the CLI via the switch’s serial interface, you can browse it. For instance, type http://192.168.1.1 in the address row in a browser, it will show the following screen and ask you inputting username and password in order to login and access authentication. The default username is “admin” and password is empty. For the first time to use, please enter the default username and password, and then click the Enter button. The login process now is completed.

PSGS-2618G web user interface :

|  | NOTE: If you need to configuration the function or parameter then you can refer the detail in the User Guide. Or you could access to the Switch and click the "help" under the web GUI and the switch will pop-up the simple help content to teach you how to set the parameters. |
| --- | --- |

PSGS-2618G web help function:

MAKING NETWORK CONNECTIONS

CONNECTING NETWORK DEVICES

The switch is designed to be connected to 10, 100 or 1000Mbps network cards in PCs and servers, as well as to other switches and hubs. It may also be connected to remote devices using optional SFP transceivers.

TWISTED-PAIR DEVICES

Each device requires an unshielded twisted-pair (UTP) cable with RJ-45 connectors at both ends. Use Category 5, 5e or 6 cable for 1000BASE-T connections, Category 5 or better for 100BASE-TX connections.

CABLING GUIDELINES

The RJ-45 ports on the switch support automatic MDI/MDI-X pinout configuration, so you can use standard straight-through twisted-pair cables to connect to any other network device (PCs, servers, switches, routers, or hubs).

See Appendix B for further information on cabling.

|  | CAUTION: Do not plug a phone jack connector into an RJ-45 port. This will damage the switch. Use only twisted-pair cables with RJ-45 connectors that conform to FCC standards. |
| --- | --- |

CONNECTING TO PCS, SERVERS, HUBS AND SWITCHES

Step1. Attach one end of a twisted-pair cable segment to the device’s RJ-45 connector.

Making Twisted-Pair Connections

Step2. If the device is a network card and the switch is in the wiring closet, attach the other end of the cable segment to a modular wall outlet that is connected to the wiring closet. (See the section “Network Wiring Connections.”) Otherwise, attach the other end to an available port on the switch.

Make sure each twisted pair cable does not exceed 100 meters (328 ft) in length.

|  | NOTE: Avoid using flow control on a port connected to a hub unless it is actually required to solve a problem. Otherwise back pressure jamming signals may degrade overall performance for the segment attached to the hub. |
| --- | --- |

Step3. As each connection is made, the Link LED (on the switch) corresponding to each port will light green (1000 Mbps) or amber (100 Mbps) to indicate that the connection is valid.

NETWORK WIRING CONNECTIONS

Today, the punch-down block is an integral part of many of the newer equipment racks. It is actually part of the patch panel. Instructions for making connections in the wiring closet with this type of equipment follows.

Step1. Attach one end of a patch cable to an available port on the switch, and the other end to the patch panel.

Step2. If not already in place, attach one end of a cable segment to the back of the patch panel where the punch-down block is located, and the other end to a modular wall outlet.

Step3. Label the cables to simplify future troubleshooting. See “Cable Labeling and Connection Records” .

.

Network Wiring Connections

FIBER OPTIC SFP DEVICES

An optional Gigabit SFP transceiver can be used for a backbone connection between switches, or for connecting to a high-speed server.

Each single-mode fiber port requires 9/125 micron single-mode fiber optic cable with an LC connector at both ends. Each multimode fiber optic port requires 50/125 or 62.5/125 micron multimode fiber optic cabling with an LC connector at both ends.

|  | WARNING: This switch uses lasers to transmit signals over fiber optic cable. The lasers are inherently eye safe in normal operation. However, user should never look directly at a transmit port when it is powered on. WARNING: When selecting a fiber SFP device, considering safety, please make sure that it can function at a temperature that is not less than the recommended maximum operational temperature of the product. You must also use an approved Laser SFP transceiver. |
| --- | --- |

Step1. Remove and keep the LC port’s rubber plug. When not connected to a fiber cable, the rubber plug should be replaced to protect the optics.

Step2. Check that the fiber terminators are clean. You can clean the cable plugs by wiping them gently with a clean tissue or cotton ball moistened with a little ethanol. Dirty fiber terminators on fiber optic cables will impair the quality of the light transmitted through the cable and lead to degraded performance on the port.

Step3. Connect one end of the cable to the LC port on the switch and the other end to the LC port on the other device. Since LC connectors are keyed, the cable can be attached in only one orientation.

Making Fiber Port Connections

Step4. As a connection is made, check the Link LED on the switch corresponding to the port to be sure that the connection is valid.

The fiber optic ports operate at 1 Gbps. The maximum length for fiber optic cable operating at Gigabit speed will depend on the fiber type as listed under “1000 Mbps Gigabit Ethernet Collision Domain”.

CONNECTIVITY RULES

When adding hubs to your network, please note that because switches break up the path for connected devices into separate collision domains, you should not include the switch or connected cabling in your calculations for cascade length involving other devices.

1000BASE-T CABLE REQUIREMENTS

All Category 5 UTP cables that are used for 100BASE-TX connections should also work for 1000BASE-T, providing that all four wire pairs are connected. However, it is recommended that for all critical connections, or any new cable installations, Category 5e or Category 6 cable should be used. The Category 5e and 6 specifications include test parameters that are only recommendations for Category 5. Therefore, the first step in preparing existing Category 5 cabling for running 1000BASE-T is a simple test of the cable installation to be sure that it complies with the IEEE 802.3-2005 standards

1000 MBPS GIGABIT ETHERNET COLLISION DOMAIN

Maximum 1000BASE-T Gigabit Ethernet Cable Length

| Cable Type | Maximum Cable Length | Connector |
| --- | --- | --- |
| Category 5, 5e or 6 100-ohm UTP or STP | 100.m (328 ft) | RJ-45 |

Maximum 1000BASE-SX Gigabit Fiber Cable Lengths

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| 62.5/125 micron multimode fiber | 160 MHz/km 200 MHz/km | 220 m (722 ft) 275 m (902 ft) | LC LC |
| 50/125 micron multimode fiber | 400 MHz/km 500 MHz/km | 500 m (1641 ft) 550 m (1805 ft) | LC LC |

Maximum 1000BASE-LX/LHX/XD/ZX Gigabit Fiber Cable Length

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| 9/125 micron single-mode fiber 1310nm | N/A | 10km (6.2 miles) | LC |
| 9/125 micron single-mode fiber 1550nm | N/A | 30km (18.64 miles) 50km (31.06 miles) | LC LC |

Maximum 1000BASE-LX Single Fiber Gigabit Fiber Cable Length

| Fiber Size | Fiber Bandwidth | Maximum Cable Length | Connector |
| --- | --- | --- | --- |
| Single-mode TX-1310nm RX-1550nm | N/A | 20km (12.42miles) | BIDI LC |
| Single-mode TX-1550nm RX-1310nm | N/A | 20km (12.42miles) | BIDI LC |

100 MBPS FAST ETHERNET COLLISION DOMAIN

Maximum Fast Ethernet Cable Lengths

| Cable Type | Maximum Cable Length | Connector |
| --- | --- | --- |
| Category 5, 5e or 6 100-ohm UTP or STP | 100.m (328 ft) | RJ-45 |

CABLE LABELING AND  CONNECTION RECORDS

When planning a network installation, it is essential to label the opposing ends of cables and to record where each cable is connected. This will allow user to easily locate inter-connected devices, isolate faults and change your topology without need for unnecessary time consumption.

To best manage the physical implementations of your network, follow these guidelines:

- Clearly label the opposing ends of each cable.

- Using your building’s floor plans, draw a map of the location of all network-connected equipment. For each piece of equipment, identify the devices to which it is connected.

- Note the length of each cable and the maximum cable length supported by the switch ports.

- For ease of understanding, use a location-based key when assigning prefixes to your cable labeling.

- Use sequential numbers for cables that originate from the same equipment.

- Differentiate between racks by naming accordingly.

- Label each separate piece of equipment.

- Display a copy of your equipment map, including keys to all abbreviations at each equipment rack.

A

TROUBLESHOOTING

Basic Troubleshooting Tips

Most problems are caused by the following situations. Check for these items first when starting your troubleshooting:

- Connecting to devices that have a fixed full- duplex configuration.

The RJ-45 ports are configured as “Auto”, that is, when connecting to the attached devices, the switch will operate in one of two ways to determine the link speed and the communication mode (half duplex or full duplex):

    - If the connected device is also configured to Auto, the switch will automatically negotiate both link speed and communication mode.

    - If the connected device has a fixed configuration, for example 100Mbps, at half or full duplex, the switch will automatically sense the link speed, but will default to a communication mode of half duplex.

Because the PSGS-2618G Switch devices behave in this way (in compliance with the IEEE802.3 standard), if a device connected to the switch has a fixed configuration at full duplex, the device will not connect correctly to the switch. The result will be high error rates and very inefficient communications between the switch and the device.

Make sure all devices connected to the PSGS-2618G Switch devices are configured to auto negotiate, or are configured to connect at half duplex (all hubs are configured this way, for example).

- Faulty or loose cables. Look for loose or obviously faulty connections. If they appear to be OK, make sure the connections are snug. If that does not correct the problem, try a different cable.

- Non-standard cables. Non-standard and miswired cables may cause network collisions and other network problems, and can seriously impair network performance. Use a new correctly-wired cable. For pinouts and correct cable wiring. A category 5 cable tester is a recommended tool for every 100Base-TX and 1000Base-T network installation.

- Improper Network Topologies. It is important to make sure you have a valid network topology. If you no longer experience the problems, the new topology is probably at fault. In addition, you should make sure that your network topology contains no data path loops.

- Check the port configuration.

A port on your Switch may not be operating as you expect because it has been put into a “blocking” state by Spanning Tree, GVRP (automatic VLANs), or LACP (automatic trunking). (Note that the normal operation of the Spanning Tree, GVRP, and LACP features may put the port in a blocking state.) Or, the port just may have been configured as disabled through software.

Troubleshooting Chart

| Symptom | Action |
| --- | --- |
| SYSTEM LED is Off | - Check connections between the switch, the power cord and the wall outlet. - Contact your dealer for assistance. |
| Link LED is Off | - Verify that the switch and attached device are powered on. - Be sure the cable is plugged into the switch and corresponding device. - If the switch is installed in a rack, check the connections to the punch-down block and patch panel. - Verify that the proper cable types is used and its length does not exceed specified limits. - Check the adapter on the attached device and cable connections for possible defects. Replace the defective adapter or cable if necessary. |

POWER AND COOLING PROBLEMS

Installation

If the System indicator does not turn on when the power cord is plugged in, you may have a problem with the power outlet, power cord, or internal power supply. However, if the unit powers off after running for a while, check for loose power connections, power losses or surges at the power outlet. If you still cannot isolate the problem, the internal power supply may be defective. Verify that all system components have been properly installed. If one or more components appear to be malfunctioning (such as the power cord or network cabling), test them in an alternate environment where you are sure that all the other components are functioning properly.

IN-BAND ACCESS

You can access the management agent in the switch from anywhere within the attached network using Telnet, a web browser. However, you must first configure the switch with a valid IP address, subnet mask, and default gateway. If you have trouble establishing a link to the management agent, check to see if you have a valid network connection. Then verify that you entered the correct IP address. Also, be sure the port through which you are connecting to the switch has not been disabled. If it has not been disabled, then check the network cabling that runs between your remote location and the switch.

|  | NOTE: The management agent accepts up to four simultaneous Telnet sessions. If the maximum number of sessions already exists, an additional Telnet connection will not be able to log into the system. |
| --- | --- |

CABLES

TWISTED-PAIR CABLE AND PIN ASSIGNMENTS

For 10/100BASE-TX connections, the twisted-pair cable must have two pairs of wires. For 1000BASE-T connections the twisted-pair cable must have four pairs of wires. Each wire pair is identified by two different colors. For example, one wire might be green and the other, green with white stripes. Also, an RJ-45 connector must be attached to both ends of the cable.

|  | CAUTION: DO NOT plug a phone jack connector into any RJ-45 port. Use only twisted-pair cables with RJ-45 connectors that conform with FCC standards. CAUTION: Each wire pair must be attached to the RJ-45 connectors in a specific orientation. |
| --- | --- |

The figure below illustrates how the pins on the RJ-45 connector are numbered. Be sure to hold the connectors in the same orientation when attaching the wires to the pins.

RJ-45 Connector Pin Numbers

10BASE-T/100BASE-TX PIN ASSIGNMENTS

Use unshielded twisted-pair (UTP) or shielded twisted-pair (STP) cable for RJ-45 connections: 100-ohm Category 3 or better cable for 10 Mbps connections, or 100-ohm Category 5 or better cable for 100 Mbps connections. Also be sure that the length of any twisted-pair connection does not exceed 100 meters (328 feet).

The RJ-45 ports on the switch base unit support automatic MDI/MDI-X operation, so you can use straight-through cables for all network connections to PCs or servers, or to other switches or hubs. In straight-through cable, pins 1, 2, 3, and 6, at one end of the cable, are connected straight through to pins 1, 2, 3, and 6 at the other end of the cable. When using any RJ-45 port on this switch, you can use either straight-through or crossover cable.

10/100BASE-TX MDI and MDI-X Port Pinouts

| Pin | MDI Signal Name |  | MDI-X Signal Name |
| --- | --- | --- | --- |
| 1 | Transmit Data plus (TD+) |  | Receive Data plus (RD+) |
| 2 | Transmit Data minus (TD-) |  | Receive Data minus (RD-) |
| 3 | Receive Data plus (RD+) |  | Transmit Data plus (TD+) |
| 6 | Receive Data minus (RD-) |  | Transmit Data minus (TD-) |
| 4, 5, 7, 8 | Not used |  | Not used |
|  |  | NOTE: The “+” and “-” signs represent the polarity of the wires that make up each wire pair. |  |  |

STRAIGHT- THROUGH WIRING

If the twisted-pair cable is to join two ports and only one of the ports has an internal crossover (MDI-X), the two pairs of wires must be straight-through. (When auto-negotiation is enabled for any RJ-45 port on this switch, you can use either straight-through or crossover cable to connect to any device type.)

You must connect all four wire pairs as shown in the following diagram to support Gigabit Ethernet.

Straight-through Wiring

EIA/TIA 568B RJ-45 Wiring Standard

10/100BASE-TX Straight-through Cable

CROSSOVER WIRING

If the twisted-pair cable is to join two ports and either both ports are labeled with an “X” (MDI-X) or neither port is labeled with an “X” (MDI), a crossover must be implemented in the wiring. (When auto-negotiation is enabled for any RJ-45 port on this switch, you can use either straight-through or crossover cable to connect to any device type.)

You must connect all four wire pairs as shown in the following diagram to support Gigabit Ethernet.

Crossover Wiring

EIA/TIA 568B RJ-45 Wiring Standard

10/100BASE-TX Crossover Cable

1000BASE-T PIN ASSIGNMENTS

All 1000BASE-T ports support automatic MDI/MDI-X operation, so you can use straight-through cables for all network connections to PCs or servers, or to other switches or hubs.

The table below shows the 1000BASE-T MDI and MDI-X port pinouts. These ports require that all four pairs of wires be connected. Note that for 1000BASE-T operation, all four pairs of wires are used for both transmit and receive.

Use 100-ohm Category 5, 5e or 6 unshielded twisted-pair (UTP) or shielded twisted-pair (STP) cable for 1000BASE-T connections. Also be sure that the length of any twisted-pair connection does not exceed 100 meters (328 feet).

1000BASE-T MDI and MDI-X Port Pinouts

| Pin | MDI Signal Name | MDI-X Signal Name |
| --- | --- | --- |
| 1 | Bi-directional Pair A Plus (BI_DA+) | Bi-directional Pair B Plus (BI_DB+) |
| 2 | Bi-directional Pair A Minus (BI_DA-) | Bi-directional Pair B Minus (BI_DB-) |
| 3 | Bi-directional Pair B Plus (BI_DB+) | Bi-directional Pair A Plus (BI_DA+) |
| 4 | Bi-directional Pair C Plus (BI_DC+) | Bi-directional Pair D Plus (BI_DD+) |
| 5 | Bi-directional Pair C Minus (BI_DC-) | Bi-directional Pair D Minus (BI_DD-) |
| 6 | Bi-directional Pair B Minus (BI_DB-) | Bi-directional Pair A Minus (BI_DA-) |
| 7 | Bi-directional Pair D Plus (BI_DD+) | Bi-directional Pair C Plus (BI_DC+) |
| 8 | Bi-directional Pair D Minus (BI_DD-) | Bi-directional Pair C Minus (BI_DC-) |

CABLE TESTING FOR EXISTING CATEGORY 5 CABLE

Installed Category 5 cabling must pass tests for Attenuation, Near-End Crosstalk (NEXT), and Far-End Crosstalk (FEXT). This cable testing information is specified in the ANSI/TIA/EIA-TSB-67 standard. Additionally, cables must also pass test parameters for Return Loss and Equal-Level Far-End Crosstalk (ELFEXT). These tests are specified in the ANSI/TIA/EIA-TSB-95 Bulletin, “The Additional Transmission Performance Guidelines for 100 Ohm 4-Pair Category 5 Cabling.”

|  | NOTE: That when testing your cable installation, be sure to include all patch cables between switches and end devices. |
| --- | --- |

ADJUSTING EXISTING CATEGORY 5 CABLING TO RUN 1000BASE-T

If your existing Category 5 installation does not meet one of the test parameters for 1000BASE-T, there are basically three measures that can be applied to try and correct the problem:

- Replace any Category 5 patch cables with high-performance Category 5e or Category 6 cables.

- Reduce the number of connectors used in the link.

- Reconnect some of the connectors in the link.

FIBER STANDARDS

The International Telecommunication Union (ITU-T) has standardized various fiber types for data networks. These are summarized in the following table.

Fiber Standards

| ITU-T Standard | Description | Application |
| --- | --- | --- |
| G.651 | Multimode Fiber 50/125-micron core | Short-reach connections in the 1300- nm or 850-nm band |
| G.652 | Non-Dispersion-Shifted Fiber Single-mode, 9/125-micron core | Longer spans and extended reach. Optimized for operation in the 1310- nm band. but can also be used in the 1550-nm band |
| G.652.C | Low Water Peak Non- Dispersion-Shifted Fiber Single-mode, 9/125-micron core | Longer spans and extended reach. Optimized for wavelength-division multiplexing (WDM) transmission across wavelengths from 1285 to 1625 nm. The zero dispersion wavelength is in the 1310-nm region. |
| G.653 | Dispersion-Shifted Fiber Single-mode, 9/125-micron core | Longer spans and extended reach. Optimized for operation in the region from 1500 to 1600-nm. |
| G.654 | 1550-nm Loss-Minimized Fiber Single-mode, 9/125-micron core | Extended long-haul applications. Optimized for high-power transmission in the 1500 to 1600-nm region, with low loss in the 1550-nm band. |
| G.655 | Non-Zero Dispersion-Shifted Fiber Single-mode, 9/125-micron core | Extended long-haul applications. Optimized for high-power dense wavelength-division multiplexing (DWDM) operation in the region from 1500 to 1600-nm. |

SPECIFICATIONS

PHYSICAL CHARACTERISTICS

PORTS

16 10/100/1000Mbps POE TP

2 100M/1Gbps RJ45/SFP Combo ports

NETWORK INTERFACE

Ports 1-16: RJ-45 connector, auto MDI/X

10BASE-T: RJ-45 (100-ohm, UTP cable; Category 3 or better)

100BASE-TX: RJ-45 (100-ohm, UTP cable; Category 5 or better)

1000BASE-T: RJ-45 (100-ohm, UTP or STP cable; Category 5, 5e or 6)

*Maximum Cable Length - 100 m (328 ft)

Ports 17-18: RJ-45 connector/ (100/1000M) SFP

BUFFER ARCHITECTURE

512KB on-chip frame buffer

AGGREGATE BANDWIDTH

36 Gbps for PSGS-2618G

SWITCHING DATABASE

8K MAC address entries

LEDS

System: POWER

TP Port: status (LINK/ACT), 10/100/1000M

SFP Port: status (LINK/ACT/SPD), 100/1000M

WEIGHT

3.3 kg (12.57 lbs)

SIZE

44(H) x 442(W) x 211.2(D)mm

TEMPERATURE Operating: 0°C to 45°C (32°F to 113°F)

HUMIDITY

Operating: 10% to 90% (non-condensing)

POWER INPUT

For PSGS-2618G: 100~240VAC, 50~60Hz

POWER SUPPLY

For PSGS-2618G: Internal: 100 to 240 VAC

, 50 to 60 Hz

POWER CONSUMPTION

235 Watts maximum

SWITCH FEATURES

FORWARDING MODE

Store-and-forward

THROUGHPUT

26.8 Mpps

FLOW CONTROL

Full Duplex: IEEE 802.3x

Half Duplex: Back pressure

MANAGEMENT FEATURES

IN-BAND MANAGEMENT

SSH/SSL, Telnet, SNMP, or HTTP

OUT-OF-BAND MANAGEMENT

RJ-45 console port

SOFTWARE LOADING

HTTP, TFTP in-band, Console out-of-band

STANDARDS

I

EEE 802.3 => 10Base-T Ethernet (Twisted-pair Copper)

IEEE 802.3u => 100Base-TX Ethernet (Twisted-pair Copper)

IEEE 802.3ab => 1000Base-TX Ethernet (Twisted-pair Copper)

IEEE 802.3z => 1000Base-X Ethernet

IEEE 802.3x => Flow Control Capability

ANSI/IEEE 802.3 => Auto-negotiation

IEEE 802.1Q => VLAN

IEEE 802.1p => Class of Service

IEEE 802.1X => Access Control

IEEE 802.1D => Spanning Tree

IEEE 802.1w => Rapid Spanning Tree

IEEE 802.1s => Multiple Spanning Tree

IEEE 802.3ad => ink Aggregation Control Protocol (LACP)

IEEE 802.1AB => Link Layer Discovery Protocol (LLDP)

COMPLIANCES

EMISSIONS

EN55022 (CISPR 22) Class A EN 61000-3

FCC Class A

CE Mark

IMMUNITY

EN 61000-4-2/3/4/5/6/8/11

EN 55024

COMPLIANCES

10BASE-T

IEEE 802.3 specification for 10 Mbps Ethernet over two pairs of Category 3, 4, or 5 UTP cable

100BASE-TX

IEEE 802.3u specification for 100 Mbps Ethernet over two pairs of Category 5 UTP cable

1000BASE-LH

Specification for long-haul Gigabit Ethernet over two strands of 9/125 micron core fiber cable

1000BASE-LX

IEEE 802.3z specification for Gigabit Ethernet over two strands of 50/125, 62.5/125 or 9/125 micron core fiber cable.

1000BASE-SX

IEEE 802.3z specification for Gigabit Ethernet over two strands of 50/125 or 62.5/125 micron core fiber cable.

1000BASE-T

IEEE 802.3ab specification for Gigabit Ethernet over 100-ohm Category 5, 5e or 6 twisted-pair cable (using all four wire pairs).

AUTO-

NEGOTIATION

Signaling method allowing each node to select its optimum operational mode (e.g., speed and duplex mode) based on the capabilities of the node to which it is connected.

BANDWIDTH

The difference between the highest and lowest frequencies available for network signals. Also synonymous with wire speed, the actual speed of the data transmission along the cable.

COLLISION DOMAIN

Single CSMA/CD LAN segment.

CSMA/CD

CSMA/CD (Carrier Sense Multiple Access/Collision Detect) is the communication method employed by Ethernet, Fast Ethernet, and Gigabit Ethernet.

END STATION

A workstation, server, or other device that does not forward traffic.

ETHERNET

A network communication system developed and standardized by DEC, Intel, and Xerox, were using baseband transmission, CSMA/CD access, logical bus topology, and coaxial cable. The successor IEEE 802.3 standard provides for integration into the OSI model and extends the physical layer and media with repeaters and implementations that operate on fiber, thin coax and twisted-pair cable.

FAST ETHERNET

A 100 Mbps network communication system based on Ethernet and the CSMA/ CD access method.

FULL DUPLEX

Transmission method that allows two network devices to transmit and receive concurrently, effectively doubling the bandwidth of that link.

GIGABIT ETHERNET

A 1000 Mbps network communication system based on Ethernet and the CSMA/ CD access method.

IEEE

Institute of Electrical and Electronic Engineers.

IEEE 802.3

Define carrier sense multiple access with collision detection (CSMA/CD) access method and physical layer specifications.

IEEE 802.3AB

Define CSMA/CD access method and physical layer specifications for 1000BASE-T Gigabit Ethernet. (Now incorporated in IEEE 802.3-2005.)

IEEE 802.3U

Define CSMA/CD access method and physical layer specifications for 100BASE- TX Fast Ethernet. (Now incorporated in IEEE 802.3-2005.)

IEEE 802.3X

Define Ethernet frame start/stop requests and timers used for flow control on full-duplex links. (Now incorporated in IEEE 802.3-2005.)

IEEE 802.3Z

Define CSMA/CD access method and physical layer specifications for 1000BASE Gigabit Ethernet. (Now incorporated in IEEE 802.3-2005.)

LAN SEGMENT

Separate LAN or collision domain.

LED

Light emitting diode used for monitoring a device or network condition.

LOCAL AREA NETWORK (LAN)

A group of interconnected computer and support devices

.

MEDIA ACCESS CONTROL (MAC)

A portion of the networking protocol that governs access to the transmission medium, facilitating the exchange of data between network nodes.

MIB

An acronym for Management Information Base. It is a set of database objects that contains information about the device.

MODAL BANDWIDTH

Bandwidth for multimode fiber is referred to as modal bandwidth because it varies with the modal field (or core diameter) of the fiber. Modal bandwidth is specified in units of MHz per km, which indicates the amount of bandwidth supported by the fiber for a one km distance.

NETWORK DIAMETER

Wire distance between two end stations in the same collision domain.

RJ-45 CONNECTOR

A connector for twisted-pair wiring.

SWITCHED PORTS

Ports that are on separate collision domains or LAN segments.

TIA

Telecommunications Industry Association.

TRANSMISSION CONTROL PROTOCOL/INTERNET PROTOCOL (TCP/IP)

Protocol suite that includes TCP as the primary transport protocol, and IP as the network layer protocol.

USER DATAGRAM PROTOCOL (UDP)

UDP provides a datagram mode for the packet-switched communications. It uses the IP as the underlying transport mechanism to provide access to IP-like services.

UDP packets are delivered just like IP packets – connection-less data grams that may be discarded before reaching their targets. UDP is useful when TCP would be too complex, too slow, or just unnecessary.

UTP

Unshielded twisted-pair cable.

VIRTUAL LAN (VLAN)

A Virtual LAN is a collection of network nodes that share the same collision domain regardless of their physical location or connection point in the network. A VLAN serves as a logical workgroup with no physical barriers, allowing users to share information and resources as though located on the same LAN.