XGS-5600 Unmanaged 10GbE Switch Series

XGS-5610TS

XGS-5610S

XGS-5610ST

XGS-5614S

XGS-5614ST

XGS-5616ST

Overview

XGS-5600 Unmanaged Switch Series is a plug-and-play 10GbE Ethernet Switch product line offering 10G copper and fiber connections in a small form factor. With the enormous growth of network traffic and network storage in recent years, 10GbE is becoming a natural upgrade for many businesses in order to keep up with their network performance and business efficiency demands.

The XGS-5600 series consists of 6 models (XGS-5610TS, XGS-5610S, XGS-5610ST, XGS-5614S, XGS-5614ST, XGS-5616ST) of which deliver 2/8 (100M/1G/10G) RJ45 ports and 2/8/10/12/14 (1G/10G) SFP+ ports. XGS-5600 provides higher bandwidth and reliability for SMB and enterprise applications.

XGS-5600 Series is ideal to provide combination of affordability and performance for entry-level networking in backbone applications and expands your enterprise network by adding local switching capacity for better scalability, so that you can support more high-bandwidth applications such as server farm, TV wall and digital signage video streaming, etc.

Key Features

- 8.6"-wide Small Form Factor Design

- All 10GbE Copper and Fiber Ports

- Auto-Negotiation for all Copper (100M/1G/10G) and Fiber(1G/10G) Ports

- Auto Fan Control for Better Reliability and Noise Reduction

- Plug-and-Play Installation with Zero Configuration

- Green Ethernet and Power Saving

Benefits

- Small Form Factor

Help you put the switch in locations that might not have sufficient room compared to other 10G switch which required larger space.

- 10GbE Interfaces

Provide superior network performance at non-blocking rate to delivers true 10GbE speed. 10GbE over CAT6a copper cabling up to 100 meters. SFP+ ports can be connected to other 10GbE ports with optical transceivers over multimode or singlemode fiber optic cabling.

Auto-negotiation senses and adjusts network speed and cabling type automatically.

- Easy to Install and Built with Affordable Ethernet Switch for SMB

It’s Plug-and-Play with zero configurations and no required ongoing administration .Build for Small and Medium Businesses provides 10G connectivity ports at a price that SMB customers can afford easily.

- Green Ethernet and Power Saving

Energy-saving capabilities save your money by lower power consumption. The smart fans feature can also reduce power consumption and noise at lower temperatures by utilizing different fan speeds for different temperatures.

Specifications

Port Configuration

| Model Name |  | Total Ports |  | Dominant Port |  | Uplink Port |  |  |  | Console |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | RJ45 100M/1G/10G | SFP+ 1G/10G | RJ45 100M/1G/10G |  | SFP+ 1G/10G |  |  |
| XGS-5610TS | 10 |  | 8 |  | -- |  | -- | 2 | -- |  |
| XGS-5610S | 10 |  | -- |  | 8 |  | -- | 2 | -- |  |
| XGS-5610ST | 10 |  | -- |  | 8 |  | 2 | -- | -- |  |
| XGS-5614S | 14 |  | -- |  | 12 |  | -- | 2 | -- |  |
| XGS-5614ST | 14 |  | -- |  | 12 |  | 2 | -- | -- |  |
| XGS-5616ST | 16 |  | -- |  | 12 |  | 2 | 2 | -- |  |

Hardware Performance

| Model Name | Forwarding Capacity (Mpps) | Switching Capacity (Gbps) | Mac Table (K) | Jumbo Frames (bytes) |
| --- | --- | --- | --- | --- |
| XGS-5610TS | 148.8 | 200 | 16 | 9216 |
| XGS-5610S | 148.8 | 200 | 16 | 9216 |
| XGS-5610ST | 148.8 | 200 | 16 | 9216 |
| XGS-5614S | 208.32 | 280 | 16 | 9216 |
| XGS-5614ST | 208.32 | 280 | 16 | 9216 |
| XGS-5616ST | 238.08 | 320 | 16 | 9216 |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Altitude |  |
| --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | Feet | Meters |
| 14 to 122 | -10 to 50 | -4 to 158 | -20 to 70 | < 10000 | <3000 |

Dimension, Weights, Humidity

| Model Name |  | Dimension (WxHxD) |  |  |  | Weight |  | Operating Humidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Millimeter |  | Inches |  | Kilograms | Pounds |  |
| All Models | 220 x 44 x 242 |  | 8.6 x 1.73 x 9.5 |  | 1.9 |  | 4.19 | 10% to 90% non-condensing |

Voltage/Frequency, Acoustic, and Compliance Information

| Model Name | FAN | Voltage/Frequency | Acoustic | Certification |
| --- | --- | --- | --- | --- |
| All Models | 2 | 1000~240V / 50~60 Hz | < 42db at 25℃ (refer to ISO 7779) | CE, FCC Class A Others by Request |