PSGC-120A

Unmanaged GbE PoE Converter

Overview

PSGC-120A unmanaged GbE PoE converter is the next generation Ethernet converter offering one gigabit per second performance. It is for Ethernet copper to fiber media conversion applications.

PSGC-120A delivers one (10M/100M/1G) RJ45 with PoE (Support 802.3 at up to 30W) port, one GbE SFP port. PSGC-120A provides high HW performance and environment flexibility for Ethernet applications.

Key Features

- IEEE 802.3ab 1000Base-T Gbit/s Ethernet over twisted pair

- IEEE 802.3z 1000Base-X Gbit/s Ethernet over Fiber-Optic

- IEEE 802.3at max. 30W Power over Ethernet

- DIP Switch to Set Configurations:

  - PoE/PoE DIS: PoE Enable(Default) or Disable

  - LFP/LFP DIS : LFP Enabled(Default) or Disabled

  - Normal/Cut-through: Normal is Bridge Mode(Default) or Cut-through Mode

Benefits

- Power over Ethernet (PoE)

The PoE converter provides PoE powering, it supports various PoE PD device types (e.g. IP-phone, IP-camera, WiFi-AP) for end users to save extra powering cable to these devices.

- Link Fault Pass-through

When Link Fault Pass-through is enabled, the UTP receiver is passed to the fiber transmitter to make the media converter appear transparent to the end devices that are connected. Using Link Fault Pass-through to indicate Far-End Fault issue occurs. Should a fault occur, the end devices have the indication of a failure available to them making trouble shooting.

- Support Fiber Connection for Long Distance Request

The fiber converter is designed to help long distance connection request. It can be used for customers to extend Ethernet networking environment.

Specifications

Port Configuration

| Model Name | Total Ports | RJ45 (10M/100M/1G) | Fiber (100M/1G) |
| --- | --- | --- | --- |
| PSGC-120A | 2 | 1 | 1 (100M/1G SFP) |

Cables

| Cable Type | Description |
| --- | --- |
| UTP | UTP Cables: - Cat. 5, or above Cat. 5e class UTP cables UTP Cable Limitations: - Max. Length up to 100M |
| Fiber | Fiber Cables: - 1000Base-SX: 50/125, 62.5/125, or 100/140µm Multi-mode - 1000Base-LX: 8.3/125, 8.7/125, 9/125 or 10/125µm Single-mode Fiber Cable Limitations: 1000Base-SX (850nm); Multi-Mode Fiber 62.5/125µm; Multi-Mode Fiber 50/125µm. Bandwidth MHz-Km; Distance; Bandwidth MHz-Km; Distance. 260; 220m; 400; 500m. 200; 275m; 500; 550m. 1000Base-LX (1310nm/1550nm); Single-Mode Fiber 9/125µm: 2Km Single-Mode Transceiver 1310nm: 10/30Km Single-Mode Transceiver 1550nm: 50Km. |

Hardware Performance

| Forwarding Capacity (Mpps) | Switching Capacity (Gbps) | Jumbo Frames (Bytes) |
| --- | --- | --- |
| 2976190 | 2 | 16K |

Environmental Range

| Operating Temperature |  | Storage Temperature |  | Operating Humidity | Altitude |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fahrenheit | Centigrade | Fahrenheit | Centigrade | 5% to 95% non-condensing | Feet | Meters |
| 32 to 122 | 0 to 50 | -4 to 158 | -20 to 70 |  | < 10000 | <3000 |

Dimension, Weights, Mounting

| Dimension (WxHxD) |  | Weight |  |  | Mounting Type |
| --- | --- | --- | --- | --- | --- |
| Millimeter | Inches | Kilograms |  | Pounds |  |
| 158x 40x 133 | 6.22x 1.57x 5.23 | < 1 | <2.2 |  | Wall Mount |

Voltage and Frequency

| Primary Power Supply |  |
| --- | --- |
| Power Characteristics | Requirement: 100 – 240 VAC, 47 –63 Hz Consumption: Max. up to 89W |

PoE Power Capacity

| Available PoE Power | Number of Ports That Support PoE(15.4W), PoE+(30W) |
| --- | --- |
| 30W | One PoE port support PoE within available PoE Power |

Certifications

| Regulatory Compliance |  |
| --- | --- |
| EMI | Comply with FCC Part 15 Class A & CE Mark Approval LVD EN 62368-1 |
| Safety | UL and others by request |

LED Indicator

| LED | Color | Function |
| --- | --- | --- |
| FX LNK/ACT | Green | Lit when fiber connection is good Blinks when packet is forwarding on fiber port |
| TP LNK/ACT | Green | Lit when TP connection is good Blinks when packet is forwarding on TP port |
| TP SPD | Green Yellow | Green lit when TP speed is 1000Mbps Yellow lit when TP speed is 100Mbps Off when TP speed is 10Mbps |
| PWR | Green | Lit when power is coming up Off when power was turned off |
| PoE | Green | Green lit when PoE feeling power is active Off when PoE feeling power was turned off |
| ALRM | Red | Red lit when PoE feeling power was disrupted (In case of over-temp or over-current) Off when PoE is powering normal |
| 15.4W | Green | Lit when PD Class Type is Class 0 - 3 |
| 30W | Green | Lit when PD Class Type is Class 4 |

Features

| DIP Switch Settings |  |
| --- | --- |
| DIP Switch (S1) for HW Configuration | S1-1 PoE/PoE DIS: PoE Enable(Default) or Disable |
|  | S1-2 LFP/LFP DIS : LFP Enabled(Default) or Disabled |
|  | S1-3 Normal/Cut-through: Normal is Bridge Mode(Default) or Cut-through Mode |

Ordering Information

| Model | Description |
| --- | --- |
| PSGC-120A | 10/100/1000Base-T to 100/1000Base-X SFP PoE+ PSE GbE converter |